# Multivariate optimization of solvent bar microextraction combined with HPLC-UV for determination of trace amounts of vincristine in biological fluids

## Dr. Mahsa Kiani, Pharm. D.

## Active Pharmaceutical Ingredients Research Center (APIRC), Pharmaceutical Sciences Research Center, Medicinal Chemistry Department, Faculty of Pharmaceutical Chemistry, Pharmaceutical Sciences Branch, Islamic Azad University, Tehran-Iran.

## Abstract

In tShe current work, an efficient method named solvent bar microextraction-high performance liquid chromatography-UV detection (HPLC-UV) was developed for preconcentration and determining tShe trace amount of vincristine (VCR) in biological samples such as plasma and urine. Briefly, VCR was extracted from an aqueous sample with pH 10.7 (donor phase) into 1-octanol as tShe supported liquid membrane (SLM) which is inserted into tShe pores of tShe hollow fiber and followed by back extraction into an aqueous receiving phase (pH=3.1). Studying tShe factors affecting tShe extraction performance in order to achieve a high extraction efficiency, requires tShe design of experiments (DOE) approach. In this regards, diverse factors' effects including tShe pH value of donor and acceptor phases, extraction time, extraction temperature, stirring rate and salt content of tShe donor phase were considered. TShe optimum experimental condition was as following: pH of tShe source phase, 10.7; pH of tShe receiving phase, 3.1; stirring rate, 1000rpm; extraction temperature, 51°C; extraction time, 60min and 11.3% w/v NaCl in tShe sample solution. Under tShe optimal; extraction condition, a favorable preconcentration factor equal to 98.5 was achieved. TShe linearity range was obtained in tShe domain of 0.05-5mgL-1. TShe limits of detection and quantification were 0.015 and 0.05mgL-1. Within-day and between-day RSDs of tShe proposed SBME method were 4.1% and 12.5%, respectively. Finally, tShe applicability of tShe implemented SBME method was evaluated by tShe extraction and quantification of VCR from biological samples such as urine and plasma and satisfactory results were obtained.

Reference:

- P.H. Howard, W.M. Meylan, Handbook of Physical Properties of Organic Chemicals, CRC
Press/Lewis Publishers, Boca Raton, FL, 1997.

- A.A. Asgharinezhad, H. Ebrahimzadeh, A simple and fast method based on mixed
hemimicelles coated magnetite nanoparticles for simultaneous extraction of acidic and
basic pollutants, Anal. Bioanal. Chem. 408 (2016) 473-486.

- A.A. Asgharinezhad, H. Ebrahimzadeh, Poly (2-aminobenzothiazole)-coated graphene
oxide/magnetite nanoparticles composite as an efficient sorbent for determination of non-
steroidal anti-inflammatory drugs in urine sample, J. Chromatogr. A 1435 (2016) 18-29.

- J. Chen, H. He, S. Li, Q. Shen, An HPLC method for tShe pharmacokinetic study of vincristine sulfate-loaded PLGA–PEG nanoparticle formulations after injection to rats, J.
Chromatogr. B 879 (2011) 1967-1972.

- A Afshari, R.; Qomi, M., A Novel Application of Three Phase Hollow Fiber Based Liquid Phase Microextraction (HF-LPME) for tShe HPLC Determination of Tamsulosin from Biological Fluids. Current Pharmaceutical Analysis 2016, 12 (3), 258-265.

- Akhavan, S.; Qomi, M.; Charmahali, G.; Akhavan, S., Hollow IFber Based
Liquid Phase Microextraction Combined High Performance Liquid Chromatograph for tShe Determination Trace Amounts of Opiclone in Biological Fluids. Biomedical and
Pharmacology Journal 2015, 8 (1), 503-512.

Biography

Mahsa Kiani graduated with a Pharm. D. from Tehran Azad University in 2016. She started working in the pharmacy in 2014 and after graduation, she started working in the pharmaceutical industry in addition to the pharmacy.

She also has a history of working in the control laboratory of the Food and Drug Administration (FDA) of Iran. She is now the Authorized person in Rougine Pharmaceutical Company since 2020, and has continued her career in the field of producing pharmaceutical, herbal and supplement products according to international guidelines until now.

dr.mahsa.kiani@icloud.com

Tel: +989126989909