Thermographic evaluation of the efficacy of a combination of diosmine, esperidine and ruscogenin (mioven700®) in patients with chronic venous disease (cvd)

Arcangelo Merla, Marco Bellapianta, Stefano Camporeale, Luciano Maria Rizzo, Paolo Valle, Nicola Amati

Aim of this study was to evaluate the clinical improvement of CVD patients using thermography,

comparing changes in thermal curves with the progressive disease’s enhancement of signs and

symptoms. A lot of scientific evidences show how flavonoids are effective for the treatment of

CVD, however in all of these the effectiveness was measured through the evaluation of signs and

symptoms with compilation of questionnaires submitted to patients. Thermography has already

been used for estimation of primitive chronic venous pathologies of the lower limbs, associated

with superficial veins insufficiency and has proved to be an innovative method, very useful in

medical diagnosis of these pathologies. The current study was performed using a Flir E753 thermal

imaging camera. Images of the lower limbs showed that patients suffered from venous disease

characterized by a higher skin temperature associated with pathological changes in veins related

with blood stasis, inflammatory states and swelling. The observational study was conducted in

four centers on patients assigned to classes C0s to C3 based on the CEAP clinical classification.

Patients were separated into two groups: those treated for 3 months with 1 cpr/die of

nutraceutical Mioven700® (Diosmine 500mg, Esperidine 90mg, Ruscogenin 100mg) and with

elastocompression (as per guidelines); and those subjected only to elastocompression. During the

enrollment phase the CEAP clinical class was assigned and the first thermographic evaluation was

performed. The parameters were re-evaluated at 30 days (T1). A final evaluation will follow at the

end of the treatment at 90 days (T2). Table of acquired images shows a comparison, for different

regions of interest, of the

cutaneous temperature distribution at the T0 and the T1 evaluation for a randomly chosen

representative subject. The thermal images are displayed in both greyscale and iron scale, in the

temperature range from 25 °C to 35 °C. The qualitative inspection of the thermal images highlights

a sharper definition of the thermal track of the vessels at T1 with respect to T0. Such process can

be better appreciated at the iron colormap presentation, and especially for the left and the

posterior views of leg. In the left view, the reduction of the thermal track for the upper vessel is

particularly evident. Such process in T1 is suggestive of a better vasomotor control, with an higher

sympathetic vascular tone, likely combined to a better flow control in the subcutaneous vascular

network, both venous and arteriolar. Thermal imaging provides an immediate visualization of the

vascular changes through their effect on the thermal imprint of the vascular network. Even this

report deals only with a preliminary qualitative analysis of a representative subject, the digital

thermal images can be further processed to get into statistical analysis of the temperature

changes along the vessel network. Preliminary results show that patients treated with a

nutraceutical Mioven700® and with elastocompression exhibit a better vasomotor control than

the group without nutraceutical, further data will be needed to confirm these results.