Abstract Template

Abstract Title: Assessment of Advanced Photocatalytic Oxidation process for Micropollutant Elimination in Municipal and Industrial Waste Water Treatment Plants

C. Vreuls(1*), S. Drot(1), S. Dircks(3),  A. Börgers(3), J. Türk(3), J. Mahy(2), C. Wolfs(2), S. D. Lambert(2)

(1) CELABOR, Research Centre, Environmental Departement, Herve, Belgium, cvr@celabor.be

(2) University of Liege, Department of Chemical Engineering – Nanomaterials, Catalysis, Electrochemistry, Liege, Belgium

(3) IUTA, Institut für Energie- und Umwelttechnik e.V, Bliersheimer Str. 58 – 60, Duisburg, Germany

Abstract (not more than 300 words)

Pharmaceuticals, personal care products, pesticides and other chemicals used for domestic purpose or industrial production are continuously discharged into wastewater and lead to global contamination of the aquatic environment. Conventional wastewater treatment removes only 20 to 50% of micropollutants. Analytical methods become more and more sensitive so that traces of organic compounds are nowadays detected in waste, surface and ground waters.

The objective of the AOPTi project is to develop and validate an innovative technology to ensure efficient elimination of different types of micropollutants and toxic effects in waste water. The process is a tertiary treatment process, which can be easily integrated into municipal and industrial WWTPs. The process is based on oxidation by ozone and a subsequent photocatalytic treatment combined with a biological active adsorption step as final finishing process. Treated water will be characterized in term of chemical transformation products and toxicity.

At the laboratory scale, 24 major micropollutants have been chosen to model waste water like pesticides, brominated compounds, pharmaceuticals, industrial chemicals, contrast agent…Their degradations are quantified by GC-MS/MS and UHPLC-MS.

Photocatalysts have been synthesized by organic sol-gel method. The best photocatalyst for the degradation of the 24 micropollutants is TiO2 doped with 2 wt.% of Ag and 10 wt.% P25. A film was then deposited inside the long alkaline-free tube for a pilot test. Experiments were conducted on 150 L of model water and on toxic industrial waste water at the exit of a WWTP. 	It is concluded that the use of a ozonation treatment followed by a photocatalytic treatment allow disrupting different micropollutants present in waste waters. Waters toxicity is evaluated before and after ozonation + UV photocatalytic treatments with daphnia magma microorganisms according to ISO Standards methods.

Keywords

Micropollutant elimination – pesticide - Photocatalyse – waste water

Professional Biography (100-150 words)

Christelle Vreuls has completed his PhD at the age of 26 years from The University of Liege and postdoctoral studies from the University of Maastricht. She is manager of the Environmenatl department in CELABOR, a research centre in Wallonia. She has published more than 10 papers in reputed journals and 3 patents.

cvr@celabor.be

Any Comments:

I prefer a talk or a poster on Tuesday May 14th, not Monday 13