Effect of Biofilm Formation on Disintegration of Benzene using Permeable Reactive Bio Barrier and Oxygen Released Compounds

Maryam Naeimi, Mahmoud Shavandi

1 Postdoc Research Fellow (Research Institute of Petroleum Industry, Iran)

2Assistant Professor (Research Institute of Petroleum Industry, Iran)

Abstract

Aromatic hydrocarbons such as benzene are ubiquitous saturated soil contaminate and cause worldwide environmental problems. They are known as major recalcitrant compounds and are also known to be carcinogenic to humans and other animals. These handicaps are caused by fractures of pipelines, leakage from underground storage facilities, leakage due to transmission, as well as accidents. Among in situ bioremediation technologies, the use of Permeable reactive barriers (PRBs) has grown as a way with high efficiency and minimal maintenance costs. The bioremediation of benzene in soils is often limited by the aeration. The use of oxygen releasing compounds (ORCs) for bioremediation in a PRB system may offer several unique advantages when dealing with contaminant plumes. Therefore, in the present study, the use of calcium peroxide nanoparticles for purification of benzene from saturated soil has been studied in permeable reactive bio barrier (PRBB). Consequently, after the synthesis and optimization of nanoparticles, calcium peroxide capsules were prepared for use in permeable reactor barriers (PRB) and studied in continuous studies. Moreover, the spatial distribution of viable benzene degrader populations and resultant biofilm formation were also monitored to evaluate the performance of the biobarrier. Biodegradation of benzene resulted in stable operation of the system for dissolved phenanthrene and pyrene during 70 days of experimentation. In addition, the results showed that encapsulating nanoparticles reduced the negative effects on the pH of the water and the present microbial population Substrate also improves pollutant removal efficiency.

Keywords— Biofilm, Benzene, Oxygen Released Compounds, Permeable Reactive Bio Barrier.

Professional Biography (100-150 words)

Maryam Naeimi has completed her PhD at the age of 31 years from Nanyang Technological University and postdoctoral studies from Nanyang Technological University and Research Institute of Petroleum Industry. The topic of her PhD was Biocementation of Sand In Geotechnical Engineering. She is a lecturer in Semnan University in the field of environmental Geotechnics. She has published more than 7 papers in reputed journals and has been serving as an editorial board member of repute.

Recent Publications (up to 5)

- Naeimi, M. and J., Chu, (2017). "Comparison of Conventional and Bio-treated Methods as Dust Suppressant," Environmental Science and Pollution Research (IF: 2.741-Q2), DOI: https://doi.org/10.1007/s11356-017-9889-1.

- Naeimi, M., J., Chu & A., Hadad, (2016). "Comparison of Chemical Source and Microbially Produced Ferrous Cations for Iron-Based Biocementation of Sand," Iranian Journal of Science and Technology Transactions of Civil Engineering (IF: 0.276), Volume 40, Issue 2, pp 149–157.

- J., Chu, V., Ivanov, J., He, M., Naeimi and Sh., Wu. (2015), Ground Improvement Case Histories: Chemical, Electrokinetic, Thermal and Bioengineering, Chapter 19: Use of Biogeotechnologies for soil improvement, Pp 571-589.

- Chu, J., V., Ivanov, M., Naeimi, V., Stabnikov, & H., Liu, (2014). “Optimization of calcium-based bioclogging and biocementation of sand.” Acta geotechnical (IF: 2.801-Q1), Volume 9, Issue 2, pp 277-285.

- Chu, J., V., Ivanov, V., Stabnikov, J., He, B., Li, & M., Naeimi, (2012). “Biocement: green building- and energy-saving material.” Advanced Materials Research (IF: 0.23), Vol. 347– 353, pp. 4051– 4054.

- Stabnikov, V., M., Naeimi, J., Chu, & V., Ivanov, (2011). "Formation of water impermeable crust on sand surface using biocement." Cement and Concrete Research (IF: 4.762-Q1), Vol. 41, pp. 1143-1149.

Email Id: naeimiM@ripi.ir