Abstract Template

Abstract Title: Hydrothermal Liquefaction of Waste Activated Sludge to Bio-oil using a Continuous Flow Reactor

Laleh Nazari, Zhongshun Yuan, Madhumita B Ray, Chunbao (Charles) Xu*

University of Western Ontario, London, ON, N6A 5B9, Canada

Abstract (not more than 300 words)

This work presents experimental and Aspen simulation results on hydrothermal liquefaction (HTL) of waste activated sludge (WAS) and co-feed of WAS and a woody biomass (rubber wood sawdust) in water-ethanol solvent (containing up to 30 wt% ethanol) in a continuous flow reactor. The continuous flow process with the (0.15:1, w/w) wood and sludge mixture at 310 oC, 10 min hydraulic retention time and 12 wt% substrate concentration, produced a yield of 32 wt% bio-oil (dry ash-free basis). Comparison of the oil yields in the continuous flow reactor with those from a batch reactor showed a slight decrease in the yields. The HTL in water-ethanol mixed solvent led to a higher yield of bio-oil with a lower molecular weight, compared to the oil produced in water alone. HTL bio-oil from WAS had high percentages of alcohols, amine and esters. The oil from (WAS + wood) were found to be rich in phenols, attributed to the degraded lignin in the woody biomass. Thermal gravimetric analysis of bio-oils showed higher volatile matters (VM) content (approx. 90 w%) from WAS than that from the co-feed of (WAS + wood) (VM content of approx. 78 wt%). Energy and mass balance calculations for the HTL process were performed using Aspen Plus software and the experimental results. Based on the simulation results, the energy requirements of the HTL process using co-feed of sludge and wood is slightly lower than that of the HTL of WAS alone. The energy demand of an HTL process could be partly compensated by utilizing the HTL by-products (e.g., char or solid residue).

Keywords — Hydrothermal liquefaction, Waste activated sludge, Woody biomass, Continuous-flow reactors

Professional Biography (100-150 words)

Dr. Charles Xu is a full Professor of Chemical/Biochemical Engineering, Director of Industrial Bioproducts Laboratory of Western University, and was the NSERC/FPInnovations Industrial Research Chair in Forest Biorefinery between 2011 and 2016. Dr. Xu earned two Ph.D. degrees, one in Metallurgical Engineering (1998) from University of Science and Technology Beijing, and the other in Chemical Engineering (2004) from The University of Western Ontario. He has acquired an international reputation in the field of biorefinery, particularly in the production of bio-energy, bio-fuels, bio-based chemicals and materials from forestry and agricultural wastes and residues. He has co-edited/authored 2 Springer books on biorefinery, published 14 book chapters nearly 160 papers in peer-reviewed journals. In accordance to Google Scholar, his work has received a total citation of 5000+ with an H-index of 36 to date. He has been serving as the co-editor-in-chief for the International Journal of Chemical Reactor Engineering (IJCRE) since 2011.

Recent Publications (up to 5)

- Xu C, Ferdosian F. Conversion of lignin into bio-based chemicals and materials, Springer-Verlag, Berlin, June 2017. ISBN 978-3-662-54959-9

- Nazari L,  Sarathy S, Santoro D, Ho D, Ray MB, Xu C*. Recent advances in energy recovery from wastewater Sludge. In: Direct Thermochemical Liquefaction for Energy Applications. Ed. L. Rosendahl, Elsevier Ltd, Woodhead Publishing, Duxford,, UK, 2018, ISBN: 978-0-08-101029-7. pp. 67-100.

- Ahmadi S, Reyhanitash E, Yuan Z, Rohani S, Xu C*. 2017. Upgrading of fast pyrolysis oil via catalytic hydrodeoxygenation: Effects of type of solvents, Renewable Energy 112: 376-382.

- Nazari L, Yuan Z, Ray MB*, Xu C*, 2017. Co-Conversion of Waste Activated Sludge and Sawdust through Hydrothermal Liquefaction: Optimization of Reaction Parameters Using Response Surface Methodology. Applied Energy 203: 1-10.

- Mahmood N, Yuan Z, Schmidt J, Tymchyshyn M, Xu C*. 2016. Hydrolytic Liquefaction of Hydrolysis Lignin for the Preparation of Bio-based Rigid Polyurethane Foam. Green Chemistry 18, 2385–2398.

Paste your Photograph here

Email Id: cxu6@uwo.ca

Any Comments: N/A