Construction of rhizofiltration system consisting of macrophyte Phragmites australis L. and Kyllinga nemoralis L.: Analysing efficiency in removing metals and pathogenic microorganisms from wastewater

ABSTRACT

This study evaluated the physicochemical parameters constructed rhizofiltration system and its efficiency for removing heavy metals and enteric pathogens from municipal wastewater. The study was conducted at an eThekwni municipal wastewater treatment plant in Kingsburgh - Durban in the province of KwaZulu-Natal, Republic of South Africa. The pilot-scale rhizofiltration unit included three different layers of substrates consisting of medium stones, coarse gravel and fine sand. The system had one section planted with Phragmites australis L. and Kyllinga nemoralis L. while the other section was unplanted and designed as the reference. Influent and effluent, plant tissue and sediment from the system were sampled bi-monthly for a period of six months and assessed for physicochemical parameters like pH, temperature, electron conductivity (EC), salinity, turbidity, biochemical oxygen demand (BOD) and chemical oxygen demand (COD) and removal of Escherichia coli, faecal coli, trace heavy metals (Cadmium, Chromium, Copper, Nickel, Lead and Zinc) over the period of 6 months using standard methods. There was an increase in pH in the planted and reference sections, respectively, was observed. The BOD and COD values reduced as compared to the influent sample. Suspended solids, turbidity and alkalinity were reduced in the planted section and in the reference sections. Considering the entire rhizofilter, heavy metals were accumulated at varying concentrations in the planted and reference section of the rhizofilter. Planted section showed a greater potential to remove heavy metals from wastewater than reference section. The entire rhizofiltration system was found to have an average pathogen removal ratio of between 45% and 98% for the various pathogens detected in the influent wastewater. Ascaris lumbricoides was reduced by 77% in the planted section and 53% in the reference section. The results obtained from this study suggest that macrophytes have a potential to remove heavy metals and pathogens from wastewater.

Keywords: wastewater treatment, Phragmites australis L.; Kyllinga nemoralis L.; heavy metals; pathogens; rhizofiltration.