Catalytic ozonation of biomethanated distillery wastewater with Fe2+ and nZVI based catalyst for biodegradability enhancement, color and toxicity reduction with biofuel production

Sameena N. Malika,b, Prakash. C. Ghosha, Atul. N. Vaidyab , Sandeep N. Mudliarb,*

aDepartment of Energy Science & Engineering, Indian Institute of Technology, Bombay, Maharashtra, India.

bCSIR – National Environmental& Engineering Research Institute, Nehru Marg, Nagpur, Maharashtra, India.

*Corresponding Author.  Email: sn.mudliar@gmail.com

Abstract

The effectiveness of O3, O3/Fe2+, and O3/nZVI processes on biomethanated distillery wastewater (BMDWW) was evaluated in terms of biodegradability index (BI) enhancement, biofuel production, COD, color & toxicity reduction. O3/nZVI pretreatment resulted higher COD, color and toxicity removal with maximum biodegradability enhancement due to more hydroxyl radical production. The O3/nZVI pretreated effluent with enhanced BI (up to 0.71) on anaerobic digestion shown biogas generation (64% methane content) in addition of 60% COD reduction. From the GC-MS analysis, 60 foremost organic compounds were predominantly detected in the distillery effluent. The disappearance of the corresponding GC-MS & FTIR spectral peaks during catalyzed ozonation process indicated the degradation or transformation of the organic compounds present in the effluent. Subsequent aerobic and anaerobic + aerobic biodegradation of pretreated effluent resulted in biodegradation rate enhancement by 1.83, 1.67: 3.5, 2.4: 4.7, 2.9 times for O3, O3/Fe2+ and O3/nZVI processes respectively.

Keywords: Biomethanated distillery wastewater (BMDWW); Ozonation; zero valent iron nanoparticles; biodegradability; anaerobic oxidation; aerobic oxidation

Professional Biography

Sameena Naaz Malik is pursuing PhD in Energy Science & Engineering Department at Indian Institute of Technology Bombay, India. She has completed her bachelor degree in Chemical engineering from Nagpur University, India. She has published 8 International papers in SCI journals.

Publications

- Sharvari C. Deshmukh, J. Senthilnath, Rashmi M. Dixit, Sameena N. Malik, Ram A. Pandey, Atul N. Vaidya, Subbaramajois N. Omkar*, Sandeep N. Mudliar*. Comparison of radial basis function neural network and response surface methodology for predicting performance of biofilter treating toluene” Journal of Software Engg & Application (2):595-603 (2012)

- Malik S N, Pugalenthi V, Vaidya A N, Ghosh P C, Mudliar S N. Kinetics of nano-catalysed dark fermentative hydrogen production from distillery wastewater. Energy Procedia 2014;54:417-30.

- T. Sarat Chandra, S.N. Malik, G. Suvidha, M.L. Padmere P. Shanmugam, S.N.    Mudliar. Wet air     oxidation pretreatment of biomethanated distillery efﬂuent: Mapping pretreatment efﬁciency in terms color, toxicity reduction and biogas generation” Bioresource Technology(158):135-140 (2014)

- S. N. Malik, T. Sarat Chandra, K.V.Padoley, S. L. Mudliar, P.D.Tembhekar,  S.N.Mudliar. Wet air oxidation induced enhanced biodegradability of distillery effluent” Journal of Environmental Management (136):132-138 (2014)

- Suvidha Gupta, Abhinav Sharma, T Sarath Chandra, Sameena N. Malik, Sandeep N. Mudliar. Ozone induced  biodegradability enhancement   and   colour reduction of  complex pharmaceutical effluent. Ozone Science & Engineering 37(6) : (2015)

- Suvidha Gupta, Abhinav Sharma, T Sarath Chandra, Sameena N. Malik, Sandeep N. Mudliar. Effect of ozone pretreatment on biodegradability enhancement and biogas generation potential from biomethanated distillery effluent. Ozone Science & Engineering 37(5) : (2015)

- Prachi Tembhekar, Kiran Padoley, Togarcheti Sarath Chandra, Sameena Malik, Abhinav Sharma, Suvidha Gupta, Ram Avatar pandey and Sandeep Mudliar. Advanced oxidative pretreatment of complex effluents for biodegradability enhancement and color reduction. Book chapter in Environmental Waste Management, 299-340: 2015, CRC press, ISBN-13:978-1-4987-2475-3

- Sameena N Malik, Prakash.C. Ghosh, Atul. N. Vaidya, Vishal Waindeskar, Sera Das, Sandeep. N. Mudliar. Comparison of coagulation, ozone and ferrate treatment processes for color COD and toxicity removal from complex textile wastewater. Water Science & Technology. DOI: 10.2166/wst.2017.062 (2017)

- Sameena N Malik, Prakash.C. Ghosh, Atul. N. Vaidya, Sandeep. N. Mudliar. Ozone pretreatment of biomethanated distillery wastewater in a semi batch reactor: mapping pretreatment efficiency in terms of COD, color, toxicity and biohydrogen generation. Biofuel .DOI: 10.1080/17597269.2017.1416521(2018)