River water quality index for Morocco using a fuzzy inference system

S. Assia1, I. Souad1, A. Mohammed1, A. Mohammed1

1. Laboratoire de Chimie Physique Générale, Université Mohammed V, Faculté des Sciences Agdal, Rabat Maroc

2. Ecole Nationale de l’Industrie Minérale « ENIM » Maroc

ABSTRACT

The aim of this work is to propose a new river water quality index using fuzzy logic. The proposed fuzzy index combines quality indicators’ prescribed thresholds extracted mainly from the Moroccan and the Quebec water legislations. The latter is reputed for its strict water quality assessment. The proposed index combines six indicators, and not only does it exhibit a tool that accounts for the discrepancy between the two base indices, but also provides a quantifiable score for the determined water quality. These classifications with a membership grade can be of a sound support for decision-making, and can help assign each section of a river a gradual quality sub-objective to be reached.

To demonstrate the applicability of the proposed approach, the new index was used to classify water quality in a number of stations along the basins of Bouregreg-Chaouia and Zizi-Rhéris. The obtained classifications were then compared to the conventional physicochemical water quality index currently in use in Morocco. The results revealed that the fuzzy index provided stringent classifications compared to the conventional index in 41% and 33% of the cases for the two basins respectively. These noted exceptions are mainly due to the big disparities between the different quality thresholds in the two standards, especially for fecal coliform and total phosphorus.

Keywords

Water quality index Surface water Fuzzy logic Bouregreg-chaouia basin Zizi-rhéris basin