Beat Sensors for Long Life IoT Applications

K. Ishibashi, R. Takitoge, D. Manyvone

Graduate School of Informatics and  Engineering

(University of Electro-Communications, Japan)

Abstract (not more than 300 words)

Long life wireless sensors are necessary to realize Trillion Sensors Universe in IoT era. We have proposed Beat Sensors, wherein the interval times of ID code transmissions correspond to physical quantity, thereby achieving such advantages as low power consumption, low cost, small size, and high accuracy of data. We have demonstrated the operations of the Beat Sensors for measuring AC power consumption of electrical appliances [1], Temperature [2], and DC current [3], and also demonstrated long communication distance by data recovery algorithm [4]. In Beat Sensors, sensor nodes uniformly consume tiny power with time, so that batteries for the sensor nodes last much longer than conventional IoT sensors using intermittent operations to reduce power consumption. Therefore, Beat Sensors can be used in such applications  as Security, Aqua-culture, and Agri-culture and so on, in which long life of sensors are inevitable.

S. Ishigaki and K. Ishibashi, "Power Beat: A Low‐cost and Energy Harvesting Wireless Electric Power Sensing Scheme for BEMS," IEEE ICBEST 2015, Singapore.

R. Takitoge, K. Ishibashi et. al.,"Temperature Beat: Persistent and Energy Harvesting Wireless Temperature Sensing Scheme, "IEEE SENSORS 2016, Orlando.

K.Ishibashi, M. Serizawa, R. Takitoge, S. Ishigaki, and T. Ishige, DC Current Beat: Wireless and Non-invasive DC Current Sensing Scheme,” Eurosensors 2017, Sep., 2017, Paris.

Takitoge M. Kishi K. Ishibashi ” Low-Power Enhanced Temperature Beat Sensorwith Longer Communication Distances by Data-Recovery Algorithm”, IEEE SENSORS 2017, Oct - Nov. 2017, Glasgow, Scotland.

Keywords— Beat sensor, IoT, Low Power, Sensor

Professional Biography (100-150 words)

Koichiro Ishibashi received the Ph.D. degree from Tokyo Institute of Technology, Tokyo, Japan, in 1985. He joined Central Research Laboratory, Hitachi Ltd. in 1985, where he investigated low-power technologies for SH microprocessors and high-density SRAMs. From 2004 to 2011, he worked for Renesas Electronics, where he developed low-power IPs, mainly for SOCs used in mobile phones. He has been a professor at The University of Electro-Communications, Tokyo, Japan, since 2011.

He has presented more than 150 papers at international conferences and has published papers in numerous journals. He was awarded R&D 100 for the development of SH4 Series Microprocessor in 1999.  Prof. Ishibashi is a member of IEICE and a Fellow of IEEE.

His current interests are IoT technologies, including ultra-low power LSI design technology, technologies for energy harvesting sensor networks and applications, and bio sensor technology.

Email Id: ishibashi@ee.uec.ac.jp

Any Comments: