Research and Development Results on Advanced Materials for Light Water Reactors and Future Works

Toshio Yonezawa  Dr. of Engineering, Tohoku University, Japan

Abstract

In the nuclear power industrial field, all materials for nuclear power plants must be materials that have sufficient experience in other industrial fields, or those that have been sufficient comprehensive test data accumulated.

Author has been studying on the many kinds of advanced materials for many components of the light water reactors, during more than 47years.  For example, Thermally Treated Alloy 690 which is applying to steam generator tubes, control rod drive mechanism tubes, etc., for pressurized water reactors (PWRs) in a world, irradiation assisted stress corrosion cracking (IASCC) resistant modified 316 stainless steel which is applying to baffle former bolts and barrel volts of core internals for Japanese PWRs, Iron base wear resistant alloy which is applying to valve sheets for Japanese PWRs, etc. And also, author has developed on new conceptual stress corrosion cracking (SCC) resistant austenitic stainless steel. The SCCs were detected in non-sensitized but heavily cold worked austenitic stainless steels for some components of boiling water reactors (BWRs). So, author idealized that the SCC should be caused by low stacking fault energy of that stainless steel, and developed the highly SCC resistant austenitic stainless steel which has high stacking fault energy, based upon the comprehensive basic study, as shown in Figure 1.

Author has studied on the comprehensive long-term corrosion tests and SCC tests, mechanical tests, weldability tests, inspectability tests, etc., for these advanced materials, to apply to the components of commercial PWRs or BWRs. Even after these advanced materials have been applied practically, author has been continuing the studies on the long-term corrosion, SCC test, etc., as pro-active research up to the end of plant life for light water reactors. That is a feature of the field of nuclear industry, and the material used for nuclear power plants must never be disposable on the spot.

Figure 1 The effect of stacking fault energy on SCC resistance for austenitic stainless steels in high temperature oxidized water at 288oC.

(SCC resistance was compared by the maximum SCC length on the fractured surface of CT specimens after exposure 1500 hours at 288 oC in 100ppm DO water.)

Recent Publications

1. T.Yonezawa, K.Fujimoto, T.Iwamura, S.Nishida (2000) Improvement of IASCC Resistance for Austenitic Stainless Steels in PWR Environment,ASTM STP 1401, pp.224-238.

2.  T.Yonezawa, K.suzuki, S.Ooki, A.Hashimoto (2013) The Effect of Chemical Composition and Heat Treatment Conditions on Stacking Fault Energy for Fe-Cr-Ni Austenitic Stainless Steel, Metallurgical and Materials Transaction A, pp5884-5896.

3.  T.Yonezawa, M.Watanabe, A.Hashimoto (2015) The Effect of metallurgical factors on PWSCC Crack Growth Rates in TT Alloy 690 in Simulated PWR Primary Water, Metallurgical and Materials Transaction A, pp2766-2780.

4.  T.Yonezawa, Y.Miyahara, H.Hanninen, M.J.Peltonen, T.Sarikka, A.Brederholm (2016) Hot Cracking Susceptibility Evaluation of Alloy 52 and 152 Weld Metals and Their Improvement, Proceedings of the 69th AnnuaI Assembly and International Conference.

5.  T.Yonezawa, M.Watanabe, A.Hashimoto (2017) The Mechanistic Study on the Stress Corrosion Crack Propagation for Heavily Cold Worked TT Alloy 690 in Simulated PWR Primary Water, Proceedings of 18th International Conference on Environmental Degradation of Materials in Nuclear Power Systems-Water Reactors.

Dr. Toshio Yonezawa

Senior Research Fellow

New Industry Creation Hatchery Center

Tohoku University

6-6-10,Aoba, Aramaki, Aoba-ku, Sendai 980-8579, Japan

TEL/FAX : +81-22-795-7522

Biography

Toshio Yonezawa has his expertise in the metallurgical evaluation and corrosion characterization for steels and nickel based alloys. He has studied on the metallurgical characteristics and corrosion properties for low alloy steels, austenitic steels and nickel based alloys during more than 47years. He originally developed more than 35 materials for nuclear reactor applications, gas turbines, etc. He has more than 35 patents for Japan, USA, European countries.

He studied in Takasago R&D Center of Mitsubishi Heavy Industries during 35years, final position in that company was Chief Staff Engineer. After retirement of that company, he moved to Tohoku University, and he has been studied and lectured during more than 12 years as Professor, and he is Senior Research Fellow of Tohoku University, now.

E-mail Address : t-yonezawa@fri.niche.tohoku.ac.jp