# Progress into energy storage technology: About the Carborundum Foam Ceramic structure and the fuel cells capacity

Karim RAGUI, PhD Lecturer, Algeria.

(Mechanical & Process Engineering Department,   USTHB University of Sciences & Technology)

### Abstract

The present research is about Carborundum Foam Ceramic (SiC) and its huge energy storage capacity in industrial approaches. Initially observed in 1824 by Jöns J. Berzelius in a synthetic diamond experience, by a parasite reaction between Carbon and Silice, this porous material is found much utilized since the industry of 1890, raison to its precious properties, such its powerful resistivity to thermal shocks and chemical oxidation, to name but a few. SiC could be produced at various specific surfaces, as between 10 and 20 m2/g. Similarly, the granulometric composition of the used mixture powder could control its porosity. For industrial applications (as catalysts), the porosity is ranged between 37% and 45%. Regarding the permeability “the key factor for most industrial applications”, a large range of the latter could be denoted. Complement to these properties, and following the Full Cells development in the recent years, such a material could be adopted against the storage-instability problem. As observed with various fluid natures, it was established that the energy storage at low double-diffusive buoyancies consists, usually, of a steady state ratio, easily controlled at pilot processes (and promotive for modest applications). By searching for a better capacity ratio, following the increase in the buoyancy impact, the fuel cell structure could become unstable, leading to a huge loss in both the process energy cost and the used materials. Facing to such a big challenge, our alternative will be Themo-Mechanical. By taking the annulus shapes as an example, the generation of a high diffusive gradient between the annulus limits could increase the buoyancy efficiency and the energy ratio, consequently. Of course critical situations will be reached, as the inner sub-flow regimes over the energy storage process. Against such a part, the full cell shape could light-up the perfect solution. In view of the fact that the nature of industrial processes is often non-linear, extremely complex and not sufficiently recognized, the development of primary conditions for energy storage optimization is of practical significance. As such, our presentation at Advanced Energy Materials Conference, at Zurich, Switzerland, will be undertaken to extend the optimum Carborundum Foam structure & buoyancy-range to make accurate information about full cells stability, and the evolution of the energy storage ratio during the operation time.

1

10-5

SiC Normalized Permeability

1/ (Di / Do)

1/ (Di / Do)

Storage Capacity

Diffusive-Energy loss

through the process

Di / Do = 20%,  Kc ≈ 5.5 10-2

Di / Do = 40%,  Kc ≈ 8.6 10-2

Di / Do = 60%,  Kc ≈ 4.6 10-3

Di / Do = 80%,  Kc ≈ 2.1 10-4

10-4

10-3

10-2

10-1

Fig.1  SiC fuel cell : between the energy storage capacity and the diffusive-energy loss at various device geometry (Di / Do) and normalized permeability (K).

Recent Publications

- Bensaid S, Marchisio D.L., fino D., Saracco G., Specchia V. (2009) Modeling of diesel particulate filtration in wall-flow traps, Chem Eng. J. 154: 211-218.

- Abdelhakim S., Nadhir L., Said A., (2018) Numerical analysis of catalytic-coated walls of an indirect internal reforming solid oxide fuel cell: Inﬂuence of catalyst coating distribution on the reformer eﬃciency, Energy Conversion and Management, 176: 357-371.

- Irani M, Alizadehdakhel A, Pour AN, Hoseini N, Adinehnia M. (2011) CFD modeling of hydrogen production using steam reforming of methane in monolith reactors: surface or volume-base reaction model, Int J Hydrogen Energy 36(24):15602–15610.

- Ragui K., Bekahla Y.K., Bennacer R., (2018) About the Heat Conduction Impact on the Thermosolutal Stability within Annulus: Silicon Carbide Ceramic Case, In book: Heat Conduction: Methods, Applications and Research. ISBN: 978-1-53614-673-8, Chapter: 5, Nova Science Publishers.

- Ragui K., Boutra A., Bennacer R., Benkahla Y.K., (2018) Progress on numerical simulation of yield stress ﬂuid ﬂows (Part I): Correlating thermosolutal coefﬁcients of Bingham plastics within a porous annulus of a circular shape, International Journal of Heat and Mass Transfer, 126: 72-94.

- Ragui K., A. Boutra, Y.K. Benkahla, (2016) On the Validity of a Numerical Model Predicting Heat and Mass Transfer in Porous Squares with a Bottom Thermal and Solute Source: Case of Pollutants Spreading and Fuel Leaks, Mechanic & Industry 17: 311.

- Ragui K., Boutra A., Benkahla Y.K., Bennacer R., (2018), Circular heat and solute source within a viscoplastic porous enclosure: The critical source dimension for optimum transfers, International Journal of Heat and Technology, 36 (2): 761-772.

- Ragui K., Boutra A., Bennacer R., Labsi N., Y.K. Benkahla, (2018), Correlating heat and mass transfer coefficients for thermosolutal convection within a porous annulus of a circular shape: case of internal pollutants spreading, Heat Mass Transfer  54: 2061.

- Ragui K., Benkahla Y.K., Labsi N., Boutra A., (2015) Natural convection heat transfer in a differentially heated enclosure with adiabatic partitions and filled with a Bingham fluid, J. Heat Transfer Research 08 : 765-783.

- Ragui K., Boutra A., Bennacer R., Benkahla Y.K., (2016) Heat and Mass Transfer into a Porous Annulus Found Between Two Horizontal Concentric Circular Cylinders. Applied Mechanics, Behavior of Materials, and Engineering Systems, Book Lect.Notes Mechanical Engineering, Springer.

Biography

Dr. Karim RAGUI progressed in Process Engineering studies to get his PhD thesis at Houari Boumediene University of Science and Technology (USTHB, Algeria), in 2016, on Numerical Simulation of Complex flows. He joined the Teachers-team of fluid mechanics and transfer phenomena, of Refining and Petrochemical fields, to become an Associate-Professor at USTHB University. His administrative responsibility covers License & Master Degrees (since 2013), as well as their Master research projects. Since 2012, he has started research activities into Double-Diffusive convection phenomenon, viscoplastic materials and its rheological behavior, SiC porous matrix, pollutants spreading and energy optimizations, in the aim to understand the various transfer behaviors through modeling and numerical simulation. Motivated by the world progress in his research-field, he visited various countries as France, China, Switzerland, Germany and Spain, to update his knowledge and improve his researches. As a result, he published two book chapters and various international papers. Karim RAGUI is a reviewer of some Elsevier & Springer journals, such as International Journal of Heat and Mass Transfer and Journal of Statistical Physics.

Email: ragui-karim@live.fr