Surface bound nanostructures of ternary r-GO/Mn3O4/V2O5 system for room temperature selectivity of hydrogen gas

M. Amarnath, K. Gurunathan*

Nano Function Materials Research Lab, Department of Nanoscience and Technology,

Science campus, Alagappa University, Karaikudi – 630 003

+91-4565-225630, kgnathan27@rediffmail.com

Abstract

Room temperature detection of highly sensitive Hydrogen (H2) gas sensing material preparation was taken as a major objective in this present work. Herein, a novel one pot hydrothermal method is proposed for the synthesis of ternary r-GO decorated Manganese oxide (Mn3O4) and Vanadium pentoxide (V2O5) nanocomposite. The significant electrical conductivity of r-GO plays an important role here to enhance the sensing property. Tunable band bending features of metal oxides over the r-GO surface makes the composite works at room temperature with high selectivity of H2. The optical, structural and morphological characteristics were analyzed by UV-Visible spectroscopy (UV-Vis), X-ray diffraction spectroscopy (XRD), Fourier transform infrared spectroscopy (FT-IR), Scanning electron microscope (SEM), and High Resolution-Transmission Electron Microscope (HR-TEM). The sensing results reveals that the present nanocomposite is selectively sensitive towards H 2 with sensitivity value of (175 %) at room temperature with response time (82 s) and recovery time   (92 s). To investigate the low detection limit gas concentration was varied in the range from 20 ppm to 50 ppm. The synergetic sensing performance and stability of this nanocomposite could be due to the formation of metal oxides with perspicuous nanostructures as heterojunction decorated over the r-GO stack layer.

Biography:

Gurunathan Karuppasamy received his Bachelor and Master of Science in Chemistry from Madurai Kamaraj University. He pursued Ph.D Chemistry – Energy from Madras University, Chennai. He visited many countries with prestigious fellowships like BOYSCAST Fellow (USA) and Brain Pool Scientist (South Korea). He worked as Research Associate in CSIR - Central Electrochemical Research Institute, Karaikudi, India. Then He moved to Centre for Materials for Electronics Technology, Pune, India as Scientist. Currently he is working as Professor and Head in the department of Nanoscience and Technology, Alagappa University, Karaikudi, India. His research area covers Photocatalysis, Hydrogen production and storage, Solar cell, Nano electronics and conducting polymer based energy devices and gas sensors.

Amarnath Marimuthu received his Bachelor of Science in Physics from Alagappa University and Master of Science in Nanoscience and Technology from Alagappa University. Currently he is pursuing his Ph. D under the supervision of Prof. K. Gurunathan in Nano functional materials lab, Alagappa University. His area of interests is two dimensional materials for room temperature gas sensing.

Presenting author details:

Full Name: Marimuthu Amarnath

Contact number: +919043560012

LinkedIn account: https://www.linkedin.com/in/amarnano/

Session name/ number:

Category: Poster