A Study on Ultra-High-Precision Magnetic Abrasive Finishing of 316L STS  Bio-Wire Material Using Eco-Friendly Magnetic Abrasive Finishing

Jeong Su Kim 1, Lida Heng 1, Sung Sik Nam 1, and Sang Don Mun 1, *

1 Division of Mechanical Design Engineering, Chonbuk National University, 664-14, Duckjin-gu, Jeonju 561-756, Republic of Korea

Corresponding Email: msd@jbnu.ac.kr; Tel.: +82-63-270-4762

Author 1: jeongsu1592@naver.com

Author 2: henglida1@gmail.com

Author 3: nam6978@naver.com

Author 4: msd@jbnu.ac.kr

Keywords: ultra-high-precision magnetic abrasive finishing, eco-friendly abrasive tools, CNT abrasive, 316L bio-wire

Abstract

This paper studies on the finishing characteristics of eco-friendly magnetic abrasive tools for 316L STS bio-wire using an ultra-high-precision magnetic abrasive finishing process. CNT is one type of the abrasive tools, which is currently used in the finishing process. But after finishing, CNT particles are found on surface of workpiece. Thus, they can give bad effects on the human health. Therefore, the eco-friendly magnetic abrasive tools are used in order to replace with CNT abrasive tools. The surface roughness Ra of workpiece before and after finishing was measured using surface roughness tester and also investigated by SEM photographs and EDS test. To understand the variations of surface roughness after ultra-high-precision magnetic abrasive finishing, the eco-friendly magnetic abrasive tools such as walnut shell abrasive, apricot abrasive, and cuttle-fish bone abrasive were applied in the finishing process. The good results of surface roughness were obtained with walnut shell abrasives at 1500 rpm, apricot seed abrasives at 1000 rpm, and cuttle-fish bone abrasives at 500 rpm. Based on the results, the surface roughness of workpiece after finishing was improved by all the abrasives types. However, the best result is found when the walnut shell abrasives were used at 1500 rpm of magnetic field rotational speed. Therefore, walnut shell abrasives and 1500 rpm of magnetic field rotational speed were fixed as the optimum condition and different vibration frequencies at (0, 5, 10 Hz) were used in order to find the optimum condition. The results showed that the best surface roughness is found with the increasing of vibrational frequencies. FEMM program was used to confirm the magnetic field distribution of magnetic field during the magnetic abrasive finishing process.

Funding: This research was funded by NATIONAL RESEARCH FOUNDATION (NRF) of Korea in 2019, (Research Project No. 2016R1D1A1B03932103).

Acknowledgement: The authors would like to appreciate the support from National Research Foundation of Korea (Project No. 2016R1D1A1B03932103) and appreciate the support from Ultra-Precision Finishing Lab for supporting the valuable discussion and guidance on the surface finishing technique.