Large-Scale High Quality Monolayer Graphene Grown Directly at 150 C via Plasma-Assisted Thermal CVD without Transfer Process

Byeong-Ju Park,1 Jin-Seok Choi,1,2 Hyunwoo Ha,1 Hyun You Kim,1 Seonhee Lee,3 Hyunjung Shin,3 Ji-Ho Eom,1 Hyung-Jin Choi,1 and Soon-Gil Yoon,1,*

1Department of Materials Science and Engineering, Chungnam National University, Daeduk Science Town, 34134, Daejeon, South Korea

2Analysis Center for Research Advancement (KARA), Korea Advanced Institute of Science and Technology, 291 Daehak-ro, Yuseong-gu, 34141, Daejeon, South Korea

3Department of Energy Science, Sungkyunkwan University, 300 CheonCheon-dong, Jangan-gu, 16419, Suwon, Gyeonggi-do, South Korea

*Corresponding author: sgyoon@cnu.ac.kr

Direct graphene growth on functional substrates via chemical vapor deposition is an attractive approach to manufacturing flexible electronic devices, as it avoids the drawbacks of transferred graphene. To fabricate flexible devices on plastic substrates, the growth temperature must be below ~200 °C to prevent substrate deformation. Here, we report the direct growth of wrinkle- and defect-free graphene on flexible substrates at low temperatures and without transfer processes. We show that defect-free graphene can be directly grown on a variety of substrates via the introduction of an ultra-thin titanium buffer layer, due to perfect lattice matching between titanium and carbon atoms. We further show that ex-situ Ti layers (TixOy) with a thickness of ~10 nm do not influence the transmittance or electrical conductivity of functional substrates. We report theoretical and experimental evidence for large-scale (4  4 cm2) high-quality graphene grown on in-situ deposited titanium-buffered substrates at 150 °C in a CH4/H2 atmosphere via plasma-assisted thermal CVD. We applied the proposed methodology to fabricate flexible and transparent thin-film capacitors with direct grown top- and bottom-graphene electrodes. These findings could pave the way to the practical exploitation of flexible electronic devices via large-scale high-quality monolayer graphene grown directly with no transfer processes.