Performance analysis of photovoltaic/thermal solar collector with cooling by CuO-water nanofluid

Husam Abdulrasool Hasan

Al-Esra'a university College, Baghdad,Iraq.

Abstract

The effect of nanoparticles (CuO) with water as its base fluid on the electrical and thermal performance of a photovoltaic thermal (PVT) collector equipped with jet impingement have been investigated. A PVT collector was tested indoor at set levels of solar irradiances and mass flow rates. The system consists of four parallel tubes and 36 nozzles that directly injects the fluid to the back of the PVT collector. The electrical performance of the PVT collector was determined based on the mean temperature of the PVT absorber plate. The CuO/water nanofluid system reported the highest electrical and thermal efficiency. The electrical, thermal, and combined photovoltaic thermal efficiencies were 12.75%, 85%, and 97.75%, respectively, at a solar irradiance of 1000 W/m2 and flow rate of 0.167 kg/s and ambient temperature of about 30 °C. Moreover, the Pmax of PVT with CuO nanofluid increased by 62.5% compared to the conventional PV module.

Fig.1 PV/Thermal collector with jet water impingement

Recent Publications

[1] Zhang X, Zhao X, Smith S, Xu J, Yu X. Review of R&D progress and practical application of the solar photovoltaic/thermal (PV/T) technologies. Renew Sustain Energy Rev 2012;16:599–617.

[26] Radziemska E. “Performance analysis of a photovoltaic/thermal integrated system. Int J Photo energy 2009:1–6.

[2] Vokas G, Christandonis N, Skittides F. “Hybrid photovoltaic/thermal system for domestic heating and cooling with a theoretical approach. Sol. Eng 2006;80(5):607–15.

[3] Tiwari A, Sodha MS. Performance evaluation of hybrid pv/thermal ater/air heating system: a parametric study. Ren. Eng 2006;31(15):2460–74.

[4] Ali Najah Al-Shamani, K Sopian, Sohif Mat, Husam Abdulrasool Hasan, Azher M Abed, MH Ruslan. Experimental studies of rectangular tube absorber photovoltaic thermal collector with various types of nanofluids under the tropical climate conditions. Energy Conversion and Management .Volume 124, 15 September 2016, Pages 528–542.

[5] Husam Abdulrasool Hasana, Kamaruzzaman Sopiana, Ahed Hameed Jaaza, Ali Najah Al-Shamania, Experimental investigation of jet array nanofluids impingement in photovoltaic/thermal collector.Volume 144, 1 March 2017, Pages 321–334

Dr. HUSAM ABDULRASOOL HASAN obtained his BSc in Mechanical Engineering from the University of technology Baghdad in 2004, MSc in Mechanical Engineering from the University of tenaga national(UNITEN)Malaysia in 2012 and PhD. in Mechanical Engineering from the solar energy solar energy institute , National University of Malaysia in 2017. He is presently the senior lecturer in Al-Esra'a university College, Baghdad,Iraq at the Department of Mechanical. He has been involved in the field of solar energy for more than 5 years. His main contributions are in solar radiation modeling, alternative material for solar absorber, solar water heating system with integrated storage system, solar desalination, solar cooling, solar assisted drying systems, grid connected photovoltaic system, combined Photovoltaic thermal or hybrid collector and solar hydrogen production system. He has published over 20 research papers in journals and conferences. Email:hussam2003hussam@yahoo.com

Phone:009647737877085

Notes/Comments: