High Performance of Patterned Sapphire Substrate Light-Emitting Diodes with Embedded Air Void by Textured Si3N4 Intermediate Layer

Name: Chao Szu Han,Ying-Hui Luo, Shih-Chieh Hsu

Tamkang University School of Chemical and Materials Engineering, New Taipei City, Taiwan

We use patterned sapphire substrate (PSS) to obtain the completed mesa-type light-emitting diodes (LEDs). It can reduce the threading dislocation densities. In addition, we use textured silicon nitride (Si3N4) to replace silicon dioxide (SiO2), which is commonly used, as a barrier to form a patterned sapphire substrate. After re-growing, we observe that the air voids exist on the top of the textured Si3N4 intermediate layer. The air voids help more light be extracted to the outside due to the total reflection, and thus enhance the light extraction efficiency of light-emitting diodes. Additionally, we use temperature dependent PL results to estimate the internal quantum efficiency (IQE) of PSS-LED and that of PSS-LED with embedded air void. While the IQE of PSS-LED is 8.87%, the other present is 11.29%. And, the PL intensity of PSS-LED with embedded air voids is 1.27 times higher than PSS-LED. Finally, we performed optical simulation by TracePro. It also reveals that the light extraction efficiency (LEE) of PSS-LED with embedded air voids is 1.44 times better than that of the PSS-LED.

Biography

Hsu, Shih-Chieh has completed his PhD at the age of 30 years from National Central University and postdoctoral studies from RCAS, Sinica. He is the Associate Professor of Department of Chemical and Materials Engineering, Tamkang University. His work focus on optoelectronic devices, semiconductor and light-emitting diodes. He has published more than 23 papers in reputed journals and has been serving as an editorial board member of repute.

Presenting author details 
Full name: Chao Szu Han
Contact number:+886-920548985
Twitter account:None
Linked In account:None

Session name/ number:Optical Materials and Plasmonics
Category: Oral presentation