Abstract

Although, as part of a general phenomenon, the piezoelectric response of Ba(Ti1-y My)O3 (M= Zr, Sn, Hf) increases in the vicinity of the orthorhombic (Amm2) –tetragonal (P4mm) and orthorhombic (Amm2)-rhombohedral (R3m) polymorphic phase boundaries, experiments in the last few years have shown that the same phase boundaries show significantly enhanced weak-field piezo-properties in the Ca-modified variants of these ferroelectric alloys, i.e., (Ba,Ca)(Ti, M)O3. So far there is a lack of clarity with regard to the unique  feature(s) which  Ca modification brings about that enables this significant enhancement. Here, we examine this issue from a structural standpoint with M = Sn as a case study. We carried out  a comprehensive comparative structural, ferroelectric and piezoelectric analysis of the Amm2 phase  in the immediate vicinity of  the P4mm - Amm2 phase boundaries of (i) Ca-modified Ba(Ti,Sn)O3, as per the nominal formula (1-x)BaTi0.88Sn0.12O3-(x)Ba0.7Ca0.3TiO3 and (ii) without Ca modification, i.e. Ba(Ti1-ySny)O3.  We found  that the spontaneous lattice strain of the Amm2 phase is noticieably smaller in the Ca-modified counterpart. Interestingly, this happens alongwith an improved spontaneous polarization  by enhancing the covalent character of the Ti-O bond. Our study suggests that the unique role of Ca-modification lies in its ability to induce these seemingly contrasting features (reduction in spontaneous lattice strain but increase the polarization).

References

[1] B. Jaffee, W. R. Cook, H. Jaffee, Piezoelectric ceramics, Academic press, London (1971).

[2] H. Fu and R.E. Cohen, Nature 403, 281 (2000).

[3]R. Guo, L.E. Cross, S.-E. Park, B. Noheda, D.E. Cox, and G. Shirane, Phys. Rev. Lett. 84, 5423 (2000).

[4] D. Vanderbilt and M.H. Cohen, Phys. Rev. B 63, 094108 (2001).

[5] D. Damjanovic  J Am Ceram Soc 88, 2663 (2005).

[6] Y.M. Jin, Y.U. Wang, A.G. Khachaturyan, J.F. Li, and D. Viehland, Phys. Rev. Lett. 91, 197601 (2003).