Evaluations of terahertz sources based on difference frequency generation of mid/far infrared NLO crystals PbM6Te10 (M = Ga, In)

Wen-Dan Cheng, Prof., State Key Laboratory of  Structural Chemistry, Fujian Institute of Research on the Structure of Matter, Fuzhou, China

Chen-Sheng Lin, PhD, State Key Laboratory of  Structural Chemistry, Fujian Institute of Research on the Structure of Matter, Fuzhou, China

Abstract (300 word limit)

Terahertz (THz) science and technology have attracted more and more attention by scientists and technicians around world due to their applications to many fields, such as material characterization and identification, medical diagnosis, health monitoring, safety check, etc. The practical applications of THz technique strongly depend on the generation of high quality THz radiation source although the THz science is versatile. In this work, we theoretically evaluate the integrate knowledge of THz source, including the phonon, photon and electron properties, based on difference frequency generation (DFG) optical process of infrared nonlinear optical materials PbM6Te10 (M = Ga, In). Specially, we employ a new formula developed by ours to calculate DFG susceptibilities of PbM6Te10 crystals. By the characteristics of materials-self properties in the THz region, we find that a larger nonlinear susceptibility or a large figure of merit will result in a large efficiency of THz source by the comparisons between the findings of PbGa6Te10 and PbIn6Te10 under the same experimental conditions, and the THz absorption will reduce the efficiency of THz source for the two materials of PbM6Te10 (M = Ga, In). The efficiency of THz source also depends on the experimental conditions. A large crystal size, strong pump intensity and small THz wavelength will gain large efficiency of THz source based on DFG process. The efficiency is a comprehensive index to evaluate the THz source based on DFG process.

Image

Figure 1. The dynamic susceptibilities of DFG process within THz zone for PbGa6Te10 (PGT) and PbIn6Te10 (PIT) crystals.

Recent Publications (minimum 5)

- Ferguson B, Zhang X-C (2002) Materials for terahertz science and technology. Nature Materials 1:26-32.

- Tonouchi M (2009) Galore new applications of terahertz science and technology. Terahertz Science and Technology. 2: 90-101.

- Avanesov S, Badikov V, Tyazhev A, et al. (2011) PbIn6Te10: new nonlinear crystal for three-wave interactions with transmission extending from 1.7 to 25 µm . Optical Materials Express 1:1286-1291.

- Luo Z-Z, Lin C-S, Cui H-H, et al. (2015) PbGa2MSe6 (M = Si, Ge): Two Exceptional Infrared Nonlinear Optical Crystals. Chem. Mater. 27:914-922.

- Cheng W-D, Lin C-S, Zhang H (2017) Theoretical evaluation of terahertz source generated from SnGa4Q7 (Q = S, Se) infrared nonlinear optical materials. ChemPhysChem DOI 10.1002/cphc.201601128.

Biography (150 word limit)

The research interests of Prof. Wen-Dan Cheng include in designing syntheses of electro-optical, frequency conversion, and thermoelectric materials, and calculations and simulations of their physical properties; modeling of structures and simulations of nonlinear optics for nano-materials, and developments of calculated methods for nonlinear optical susceptibilities based on different optical processes at molecular and solid states..Recently, Cheng’s work focus on the evaluation of terahertz source generating from optical frequency mixing of mid/far infrared nonlinear optical materials, in views of calculating properties of phonon, photon, and electron based on first-principal method.

Email: cwd@fjirsm.ac.cn

Notes/Comments: