Title: Electrical investigations of PbTiO3 ceramics with Pb/Ti contents fabricated through solid state sintering reaction method.

Nasira Sareecha

The Islamia University of Bahawalpur, Pakistan

Polycrystalline PbTiO3 ceramics were fabricated through solid state sintering reaction method at Pb/Ti molar ratio of x = 1.00, 0.98 and 0.94. Keeping the technological position of PbTiO3 ceramics for variety of applications; electrical investigations of crack free sintered PbTiO3 ceramics were struggled under varying processing parameters in the wide spread spectrum of temperature from 40 -700°C at 1k Hz frequency. Stoichiometry and sintering regime strongly influenced the phase transition (TC) of PbTiO3 ceramics; compositions 1.00 and 0.98 showed sharp phase transition predominantly at 490◦C. Impedance spectroscopy revealed dielectric anomalies with a relaxor like behavior at higher temperatures. The temperature dependence of alternative current conductivity (σac) confirmed the presence of ferroelectric to paraelectric phase transition. At room temperature, resistivity (ρ25) increased with increasing titanium contents. All specimens showed semiconductor behavior with negative temperature coefficient of resistivity (NTCR) characteristics; expanding drift mobility,  μd  through increasing temperature concerted the rise in conductivity. The bulk conductivity followed the Arrhenius law with Ea= 2.3265- 2.6269, 0.8302- 0.7246 and 1.7665-0.3889 eV which can be attributed to the ionic conduction governed by V''Pb, V'O and V''O vacancies. Dielectric studies at PbTiO3 ceramics fabricated with optimal 0.98 composition has potential application for high temperature applications.

Biography

Nasira Sareecha is a Ph. D scholar, works as an assistant professor at physical Chemistry laboratory; The Islamia university of Bahawalpur, Pakistan. Recently, she has entered the field of material science and ceramic engineering. Her field of interest is polycrystalline ferroelectric bulk ceramics (BaTiO3 and PbTiO3). She did a work on number of publications. She made innovative contributions while investigating dielectric and electrical properties of the materials under the widespread spectrum of temperature. She has 5 publications. The 3 are published in the renowned journals of material science; the others are in the process of publications.

Presenting author details

Full name: Nasira Sareecha

Cell No.: 00923017185509

Category: Oral presentation