The role of CNTs on the characteristics of mechanically alloyed and spark plasma sintered CNT-Fe nanocomposites

Jongsuk Nam, Department of Materials Science and Engineering, Myongji University, Republic of Korea

Chansun Shin, Department of Materials Science and Engineering, Myongji University, Republic of Korea

Abstract (300 word limit)

Metal composites reinforced by carbon nanotube (CNT) often show improved mechanical properties along with various interesting properties associated with the interfaces between CNTs and metal matrix. For example, ion irradiation experiments on CNT-aluminum composite demonstrate that the CNT-metal interfaces can act as efficient defect recombination sites and as a result reduce void generation and radiation hardening [1]. Most of the metal matrixes studied so far are non-ferrous metals, e.g., Al, Cu, Ni etc. [2], and there are only few studies on CNT-ferrous alloy. A recent research paper concerning the fabrication of 304LSS–CNT composites using mechanical alloying combined with hot-pressing and melting has also been found [3]. Recently, we fabricated and investigated a 304 stainless steel and carbon nanotube (304SS-CNT) composite with an aim to study its microstructures and high-temperature tensile properties. 304SS powders were mixed with carbon nanotubes using ball milling and consolidated using the spark plasma sintering technique. Tensile specimens made from the consolidated samples of 304SS-CNT were tested in a temperature range from 299 K to 773 K. The yield strength and the work hardening of the 304SS-CNT sample were found to be higher than those of a sample fabricated from 304SS without carbon nanotubes for all tested temperatures. 304SS-CNT sample has a microstructure significantly different from the 304SS sample, e.g. reduced grain size and many small cuboidal particles. Composition analysis using energy-dispersive spectroscopy revealed that the cuboidal particles are chromium carbides, and the chromium content is reduced in the 304SS-CNT matrix. In this study, we extended our previous work to CNT-Fe matrix composites. We will present the role of CNTs on the formation of microstructures and mechanical properties of pure Fe and ferrous alloys.

Recent Publications (minimum 5)

- So KP, Kushima A, et al. (2016) Dispersion of carbon nanotubes in aluminum improves radiation resistance. Nano Energy 225:319-327.

- Bakshi SR, Lahiri D, Agarwal A (2010) Carbon nanotube reinforced metal matrix composites – a review. Int Mater Rev 55:41-64.

- Massey C, Umanzor M, Vasudevamurthy G (2016) Thermal and mechanical properties of mechanically alloyed 304LSS-CNT metal matrix composites. J Comp Mater 51:1019-1028.

4. Cho H-h, Lim S, Jin H-H, Kwon J, Hong S-J, Shin C (2018) Microstructures and high-temperature tensile properties of mechanically alloyed and spark plasma-sintered 304SS-CNT composites. J Comp Mater DOI:10.1177/0021998317753887

Biography (150 word limit)

Jongsuk Nam is a graduate student at Department of Materials Science and Engineering, Myongji University in Republic of Korea. His expertise is in manufacturing composites utilizing ball milling and spark plasma sintering. He is expected to finish his Master’s degree in February, 2019.

Email: chansun.shin@gmail.com

Notes/Commen