# Improvement of CFRP electrical conductivity by applying                            nano-enabled products containing carbon nanotubes.

Anna Boczkowska, Professor, Warsaw University of Technology, Poland

Paulina Latko-Durałek, PhD Eng, Warsaw University of Technology, Poland

### Abstract

### Due to their low weight and high mechanical performance, Carbon Fiber Reinforced Polymer (CFRP) is used in the aerospace, automotive and defense industries. In comparison to metal parts, they can reduce total mass but cannot reduce electrostatic discharge or protect against lightning strikes. Therefore, a new challenge is to find a way to improve the electrical conductivity of CFRP, especially throughout its thickness. The most promising idea is to use carbon nanotubes (CNTs), which show not only high electrical conductivity but also good thermal conductivity and mechanical strength, while maintaining low density.

### There are different approaches to introducing CNTs into CFRP in manufacturing. One such approach is mixing the resin with CNT powder before performing the infusion. However, the significant increase of resin viscosity in the presence of CNTs makes the infusion process difficult. Another way is to bond CNTs covalently onto carbon fabrics.

### A more convenient way is to apply nano-enabled products such as thermoplastic nonwoven fabrics containing CNTs (CNT-doped veils). The first manufacturing method involves the production of fibers and their thermal bonding; the second one way is direct melt blowing of thermoplastic polymers doped with CNTs. Implementation of both types of nonwoven fabrics in CFRP as interlayers by pre-preg and resin infusion results in good impregnation. When compared to the reference CFRP, the addition of CNTs increases the electrical volume conductivity throughout the panel thickness by as much as 350%. The obtained results are very promising for the further application of CFRP with CNTs as novel, lightweight and conductive structures for the replacement of metallic parts in many industrial sectors.

### Acknowledgments

The research leading to these results received funding from the European Union Horizon 2020 Program under Grant Agreement n° 646307, and from grant no DOB-1-3/1/PS/2014 from the Polish National Centre for Research and Development.

Recent Publications

- Gaztelumendi I, Chapartegui M, Seddon R, Florez S, Pons F, Cinquin J. Enhancement of electrical conductivity of composite structures by integration of carbon nanotubes via bulk resin and/or buckypaper films. Compos Part B Eng. 2017;122:31-40. doi:10.1016/j.compositesb.2016.12.059

- Ballocchi P, Fogel M, Parlevliet P, Walsh R. Enhancement of electrical conductivity of aerospace structures by incorporation of CNT doped carrier materials into dry preforms. Master Cl Cent Natl Compos. 2013;3(6):511. doi:10.2217/pmt.13.66

- Lopes PE, van Hattum F, Pereira CMC, et al. High CNT content composites with CNT buckypaper and epoxy resin matrix: Impregnation behaviour composite production and characterization. Compos Struct. 2010; 92(6):1291-1298. doi:10.1016/j.compstruct.2009.11.003

- Akcin Y, Karakaya S, Soykasap O. Electrical, Thermal and Mechanical Properties of CNT Treated Prepreg CFRP Composites. Mater Sci Appl. 2016;7(9):465-483. doi:10.4236/msa.2016.79041

- Islam MS, Deng Y, Tong L, et al. Grafting carbon nanotubes directly onto carbon fibers for superior mechanical stability: Towards next generation aerospace composites and energy storage applications. Carbon N Y. 2016;96:701-710. doi:10.1016/j.carbon.2015.10.002

- Paulina Latko, Anna Boczkowska. Fabrication of carbon nanotube- doped veils. In Smart Intelligent Aircraft Structures (SARISTU): Proceeding of the final project conference. Piet Wölcken, Michael Papadopoulos (red), Springer, ISBN: 978-3-319-22412-1, DOI10.1007/978-3-319-22413-8, str.815-825 (2015)

- http://www.nanocomposites-hub.com/

Biography

Anna Boczkowska is an associate professor at the Faculty of Materials Science and Engineering, Warsaw University of Technology in Poland. She graduated from the same faculty in 1989 and received her PhD degree in 2000 and D.Sc degree in 2011. Scientific experience is related to the processing and structure of polymer matrix composites, nanocomposites and smart materials. Industrial background of over 15 years in the development and application of polymers and composites. Member of many international organizations (e.g. ACS, AAAS, SPIE) and author of numerous scientific publications, books and patents.

Email anna.boczkowska@inmat.pw.edu.pl