Electroless nickel plating onto Plexiglas® through simple covalent grafting of vinylpyridine seed layer

Ngoc Pi Vu a, Xuan Truong Duong b,c, Viet Anh Ly a, Duy Cuong Nguyen a, Minh Duc Tran a, Quang The Phan a, Marek Balazinski b, Le Thanh Son d, Gul Zeb d, Xuan Tuan Le a,e

- Thai Nguyen University of Technology; 3-2 Street, Thai Nguyen City, Vietnam.

- Department of Mechanical Engineering, Ecole polytechnique de Montréal, Montréal, QC, H3C 3T5, Canada.

- iCantech Innovation Inc., 167 Rue Desrochers, J2H 0R2, Granby, QC, Canada.

- Faculty of Chemistry, VNU University of Science, 19 Le Thanh Tong str., Hanoi, Vietnam.

- MiQro Innovation Collaborative Centre (C2MI)-Teledyne Dalsa, 45 boul. de l'Aéroport, Bromont, QC, J2L 1S8, Canada.

# Abstract

The development of facile and environment friendly strategies to metallize the surface of insulating polymeric substrates is of paramount important. An acceptable film adhesion with the polymeric substrate usually requires a chemical pretreatment of the polymer surfaces. Here we report a one-step process of covalent grafting of vinylpyridine groups on the surface of commercial Plexiglas® (polymethylmethacrylate) employing diazonium-based aqueous chemistry, without requiring any surface pretreatment step. Prior to the electroless nickel plating, these grafted vinylpyridine groups can be easily activated by simply immersing the substrate into an acidic palladium chloride solution. This results in stable electrostatic interaction between the protonated pyridine functionalities and the palladium complexes. Thanks to the strong attachment to the substrate, this vinylpyridine seed layer also offers excellent adhesion between the Plexiglas® substrate and the electrolessly deposited nickel-boron film. A sizable patterned Plexiglas® sample has been taken as a model considering its commercial potential and metallized. X-ray photoelectron spectroscopy is carried out to elucidate the mechanism of surface functionalization with pyridine groups as well as the activation of the grafted vinylpyridine seed layer. Comprehensive characterization of the electrolessly deposited metallic film is then performed by means of SEM, AFM, EDX and XPS techniques.

Key-words: vinylpyridine grafting, diazonium chemistry, chemisorption of palladium activator, nickel-boron alloy, thin film adhesion