Surface texturing with slanted silicon nanopillars to reduce its optical reflectivity

Jun-hyun Kim, Department of Chemical Engineering and Department of Energy Systems Research, Ajou University, Suwon 16499, South Korea

Chang-Koo Kim, Department of Chemical Engineering and Department of Energy Systems Research, Ajou University, Suwon 16499, South Korea

Abstract

Antireflective surfaces are useful in many applications such as solar cells, light emitting diodes, and optical lens systems. Coating antireflection layers is one of the ways to improve light trapping and decrease the optical reflectivity from the surface. By controlling the refractive index of the coating layer, the destructive interference of light minimizes the reflection. However, the use of hetero-materials can limit the thermal stability and eventually the long-term reliability. The surface texturing is another way of producing antireflective surfaces. Surfaces with silicon pillars in various shapes have been shown to exhibit low reflectivity compared with bare surfaces. These pillars are mostly vertical. If the pillars are tilted from the surface normal, more light would be trapped between the pillars. This would result in lower reflectivity of the surfaces. The surface can be textured through either wet- or dry-etching. Wet chemical etching is very simple to operate, but only crystalline substrates can be used to obtain slanted structures in wet chemical etching. Dry etching using gaseous plasmas offers high aspect ratio structures due to its anisotropic etching characteristics. In a conventional plasma etching process, a sheath is formed along the surface of a substrate, irrespective of its angle. Therefore, simply tilting of a sample is not adequate for obtaining slanted etch profiles during conventional plasma etching. In this study, slanted silicon pillars at arbitrary angles were fabricated using the slanted plasma etching technique. Optical analysis of surfaces with silicon pillars was made to investigate their antireflection properties. It was shown that the reflectance of the surface with Si pillars was affected by the aspect ratio as well as the opening area of the pillars.

Recent Publications

- Jun-Hyun Kim, Jeong Geun Bak, Kangtaek Lee, Chang-Koo Kim (2018) Control of the electrical resistivity of Ni-Cr wires using low pressure chemical vapor deposition of tin. Applied Surface Science 429:134-137.

- Jun-Hyun Kim, Sung-Woon Cho, Chang-Koo Kim (2017) Angular dependence of Si3N4 etching in C4F6/CH2F2/O2/Ar plasmas. Chemical Engineering & Technology 429:2251-2256.

- Jun-Hyun Kim, Sung-Woon Cho, Chang Jin Park, Heeyeop Chae, Chang-Koo Kim (2017) Angular dependences of SiO2 etch rates at different bias voltages in CF4, C2F6, and C4F8 plasmas. Thin Solid Films 637:43-48.

- Sung-Woon Cho, Jun-Hyun Kim, Hae-Min Lee, Heeyeop Chae, Chang-Koo Kim (2016) Superhydrophobic Si surfaces having microscale rod structures prepared in a plasma etching system. Surface & Coatings Technology 306:82-86.

- Jun-Hyun Kim, Sung-Woon Cho, Doo Won Kang, Kyung Mi Lee, Chang Yong Baek, Hae-Min Lee, Chang-Koo Kim (2016) Electrical, structural, and morphological characteristics of dopantless tin oxide films prepared by low pressure chemical vapor deposition. Science of Advanced Materials 8:117-121.

Jun-Hyun Kim received the B.S. and M.S. degrees in chemical engineering from Ajou University, Suwon, South Korea, in 2011 and 2013, respectively. He is currently a Ph.D candidate in chemical engineering from Ajou University. He has expertise in plasma processing, especially plasma etching.

Email: kairi0401@ajou.ac.kr

Chang-Koo Kim received the B.S. and M.S. degrees in chemical engineering from Seoul National University, Seoul, South Korea, in 1992 and 1995, respectively, and the Ph.D. degree in chemical engineering from the University of Houston, Houston, TX, USA, in 2000. He was a Senior Process Engineer with Novellus Systems Inc. from 2001 to 2002. Since 2002, he has been a Professor with the Department of Chemical Engineering and the Department of Energy Systems Research, Ajou University. His research interests include the plasma etching processes and the deposition of capping/barrier layers for Cu interconnection.

Email: changkoo@ajou.ac.kr