# Growth and electrochemical characterization of graphene nanowalls and carbon nanotubes

Roger Amade1,2, Arevik Musheghyan-Avetisyan1,2, Joan Martí-González1,2, Fernando Pantoja-Suárez1,24, Islam Alshaikh1,2, Shahzad Hussain1,2, Jose Luís Andújar1,2, Esther Pascual1,2, Enric Bertran1,2, Ángel Pérez del Pino3, Eniko György3,

1ENPHOCAMAT group, Departament de Física Aplicada, Universitat de Barcelona,  Spain

2Institute of Nanoscience and Nanotechnology (IN2UB), Universitat de Barcelona, Spain

3Instituto de Ciencia de Materiales de Barcelona, Consejo Superior de Investigaciones Científicas (ICMAB-CSIC), Spain

4ENPHOCAMAT group, Departamento de Materiales, Facultad de Ingeniería Mecánica, Escuela Politécnica Nacional, Ecuador

### Abstract (300 word limit)

Electrochemical double layer capacitors (EDLC) or supercapacitors exhibit higher specific capacitance than conventional electrolytic capacitors due to their increased surface area and short distance between positive and negative charges at the electrode/electrolyte interface. Because of their high electric conductivity, chemical inertness, thermal and mechanical stability, carbon-based electrodes are the preferred material of choice in supercapacitor applications. In particular, carbon nanostructures such as carbon nanotubes (CNTs), with a high specific surface area may increase the capacitance upto about 100 F/g. Recently, graphene nanowalls (GNWs) are being the focus of research in different areas due to their outstanding properties [1-3]. GNWs can be described as self-assembled, vertically-standing, few-layered graphene sheet nanostructures. The growth mechanism of these nanostructures is still not clear, but recent results indicate that they grow virtually on every substrate that withstand the synthesis temperature (around 600ºC) without the need of a catalyst. Thus, this new material have promising features that may improve performance of energy storage devices like supercapacitors or lithium ion batteries.

Surface functionalization of these nanostructures by means of plasma treatments or deposition of metal oxides may further improve their pseudocapacitance and electrochemical performance. This study explores the growth of GNWs and their supercapacitive properties grown under different conditions, and compares the results with those obtained for CNTs [4,5].

### Image

Fig 1. Top view of Graphene Nanowalls deposited on silicon wafer by means of ICP-CVD (Inductively Coupled Plasma-Chemical Vapor Deposition).

Recent Publications (minimum 5)

- Davami K, Shaygan M, Kheirabi K, Zhao J, Kovalenko DA, Rümmeli MH, Opitz J, Cuniberti G, Lee JS, Meyyappan M(2014) Synthesis and characterization of carbon nanowalls on different substrates by radio frequency plasma enhanced chemical vapor deposition, Carbon Journal 72: 372-380..

- Song X, Liu J, Leyong Y, Yang J, Fang L, Shi H, Du C, Wei D (2014) Direct versatile PECVD growth of graphene nanowalls on multiple substrates, Materials Letters Journal 137:25-28.

- Bo Z, Yang Y, Chen C, Yu K, Yana J, Cena K (2013) Plasma-enhanced chemical vapor deposition synthesis of vertically oriented graphene nanosheets, Nanoscale J. 5:5180-5204.

- Pérez del Pino A, György E, Alshaikh I, Pantoja-Suárez F, Andújar JL, Pascual E, Amade R, Bertran-Serra E (2017) Laser-driven coating of vertically aligned carbon nanotubes with manganese oxide from metal organic precursors for energy storage, Nanotechnology 28:395405:1-9.

- Hussain S, Amade R, Moreno H, Bertran E (2014) RF-PECVD growth and nitrogen plasma functionalization of CNTs on copper foil for electrochemical applications, Diamond and Related Materials 49:55-61.

Biography (150 word limit)

Roger Amade has his expertise in the synthesis of carbon nanostructures using chemical vapor deposition (CVD) and plasma-enhanced chemical vapor deposition techniques on different substrates, and their electrochemical characterization as electrodes for energy storage and production devices. In particular his research is focused in the development of new nanostructures for supercapacitors, lithium-ion batteries and microbial fuel cells. He is currently associate professor at the Department of Applied Physics (University of Barcelona).

Email: r.amade@ub.edu

Notes/Comments: