# Core-Shell Structure Nanomaterials Ceria-Nickel Catalyzes Carbon Oxidation in a Molten Hydroxide Direct Carbon Fuel Cell

Xiaofeng Li, Yanfang Gao *

College of Chemical Engineering, Inner Mongolia University of Technology, Hohhot, P.R. China

The molten hydroxide direct carbon fuel cell uses molten hydroxide (NaOH or KOH) as the electrolyte which is contained within a metallic container which also acts as a cathode. A carbon rod made from graphite or coal derived carbon is dipped into the electrolyte and used as both the fuel and anode of the cell. Such a system was considered to have a number of advantages over the molten carbonate electrolyte DCFC. The advantages of using sodium hydroxide electrolyte include high ionic conductivity (especially when associated with water) [1,2], high reactivity towards carbon [3] and a low melting point [4].

Nanostructured ceria-nickel is an innovative catalyst concept with a configuration that is inverted relative to that of a conventional supported catalyst, which maybe a “sushi”-type structure. Carbon oxidation is supposed to be efficiently promoted by maximizing the nickel-ceria interface to activate oxygen species, increasing the contact between ceria and carbon to facilitate spillover of active oxygen onto the carbon at large distances, and covering nickel with ceria particles to prevent nickel sintering.The nanocatalyst with special structure was mixed with lignite pyrolysis carbon to enhance the oxidation activity of anodic carbon fuel, and the mixture is acted anodic carbon fuel for molten hydroxide direct carbon fuel cell.

References

[1] Pesavento PV, Carbon-Air fuel cell. US Patent office, patent no. 6, 200, 697,2001.

[2] Eberz A , Franck E U . High Pressure Electrolyte Conductivity of the Homogeneous, Fluid Water-Sodium Hydroxide System to 400°C and 3000 bar[J]. Berichte der Bunsengesellschaft für physikalische Chemie, 1995, 99(9):1091-1103.

[3] Claire Hérold, Albert Hérold, Lagrange P . Ternary graphite intercalation compounds associating an alkali metal and an electronegative element or radical[J]. Solid State Sciences, 2004, 6(1):125-138.

[4] FTsalteFACT Salt Phase Diagrams, Centre for Research in Computational Thermochemistry (CRCT), Facility for the Analysis of Chemical Thermodynamics FACT), 2012. http://www.crct.polymtl.ca/fact.

Biography

Yanfang Gao has completed her PhD at the age of 30 years from Fukui University and postdoctoral studies from Tsinghua University of Chemistry. She is a professor at the department of Inner Mongolia University of Technology, a tutor of a phD student. She has published more than 30 papers in reputed journals.

Presenting author details

Full name: Xiaofeng Li

Contact number: +86 13081512254

Category: ( Poster presentation)