THE USE OF GREEN SYNTHESIZED SILVER NANOPARTICLES ON LEATHER

*Kashif Ahmed, Niaz Ahmed and Irfat Ashraf

Department of Chemistry, N.E.D. University of Engineering and Technology (75950),

Karachi, Pakistan

Key words: Conocarpus erectus, Azadirachta indica, Nerium indicum, Silver Nanoparticles, silver treated leather.

Abstract

The present work shows the determination of total phenolic compounds in plant leaves extract of Azadirachta indica, Conocarpus erectus and Nerium indicum and then Silver nanoparticles (Ag NPs) were prepared by using plant leaf extract as reductant and their application on leather. Ag NPs were characterized by SEM (Scanning Electron Microscope) and XRD (X-Ray Diffractometer) methods. The size of Ag NPs size was assessed in the limit of 30-80 nm. UV-Visible spectroscopy and SEM was used to confirm that Ag NPs were set down on the collagen fibers surface and inner side of the skin collagen matrix of fibers (leather). The antibacterial and antifungal effect of Ag NPs was assessed by general microbiological test for seven days. The result shown power full strength of silver Nano particles against bacterial or fungal attack for long duration of time. This permits us to deliberate the assimilation of these Nano particles into leather as a feasible substitute of other than commercially available expensive products in order to gain leather with enhanced antimicrobial properties.

___________________________________________________________________

*Corresponding Author:kashif25473@yahoo.com +923012275358