Title: Aerodynamic Heating Analysis of Hypersonic Flow over Blunt-Nosed Bodies Using Computational Fluid Dynamics

Aakash Chhunchha

California State University, Long Beach, CA, 90840, USA

The qualitative aspects of hypersonic flow over a range of blunt bodies were analyzed by Fraasa and Wisenbaker (1957) at a Mach number of 5.8. Their experiments showed that the curvature of a body’s geometry in the sonic region predominantly dictates the bow shock shape and its stand-off distance from the body, while the surface pressure distribution depends on both the sonic region and on the local body shape. The present study is an extension to analyze the hypersonic flow characteristics over several blunt-nosed bodies using modern CFD tools to determine the shock shape and its effect on the heat flux around the body.

A total of 4 blunt nosed models with cylindrical afterbodies were analyzed for a flow at a Mach number of 10 corresponding to the standard atmospheric conditions at an altitude of 50 km. The nose radii of curvature of the models range from a hemispherical nose to a flat nose.

Appropriate numerical models and the supplementary convergence techniques that were implemented for the CFD analysis are thoroughly described.

The flow contours are presented highlighting the key characteristics of shock wave shape, shock stand-off distance and the sonic point shift on the shock. The variation of heat flux, due to different shock detachments for various models, is comprehensively discussed. It is observed that more the bluntness of the nose radii, farther the shock stands from the body; and consequently, lesser is the surface heating at the nose. The results obtained from the CFD analysis are compared with approximated theoretical engineering correlations. Overall, a satisfactory agreement is observed between the two.

Bibliography

Aakash Chhunchha has completed his graduate thesis at the age of 25 and earned his Master’s degree from California State University, Long Beach. He is a research oriented and has previously presented a research paper at International Astronautics Congress (IAC) in 2013 in Beijing, China. Presently, he is involved with carrying out research in the field of high speed aerodynamics focusing on thermal aspects of atmospheric entry vehicles.

Presenting author details

Full name: Aakash Chhunchha

Contact number: +1(562)-537-2894

Linked In account: https://www.linkedin.com/in/aakash-chhunchha-3277949b/

Session name/number: Recent development in Astronautics

Category: Oral presentation