A New Concept in Generation of 3-D Low Drag Aerofoil Shapes

# Air Commodore (retd) SC Gupta*

# Department of Aeronautical and Aerospace Engineering

# MVJ College of Engineering, Bangalore

# _____________________________________________________________________

# * Professor & Head

# Mob no :  9901163263

# E mail: satish_chander_gupta@yahoo.co.in

# Abstract

Optimization and Inverse design techniques are used to develop a new concept in generation of high lift/drag ratio wings while constraining the given lift. Three computer codes have been used for this purpose. Wing is designed for a target pressure distribution which is obtained for incompressible flow. The target pressure distribution is obtained for minimum induced drag generating camber. The inverse design is then attempted to generate a camber at higher compressible Mach number value to support this incompressible pressure distribution. Substantial gain in lift/drag ratio is obtained. The thickness effects of aerofoil are superimposed and Mach number is increased to see the occurrence of shock. The new aerofoil thus designed is compared with the aerofoil that is otherwise generated for minimum induced drag at compressible Mach number directly without this inverse technique. Transonic flow nature of two wings is compared. There is gain in critical Mach number value for the new design case.

Flow chart below describes the physics ( Figure-1 to refer). In this way the induced drag due to compressibility is minimized to the lowest possible level. Three computer codes are utilized. Programs are developed in FORTRAN language. Gfortran complier of Fedora operating system is used.  The optimal warp is broken into twist and camber. The washout at root is used to off-load angle-of-attack. Residual twist is worked out thereafter by reducing washout at root from twist. This process is shown in Figure-2. New airfoil is developed with residual twist and camber combined with root washout and thickness effects superimposed. NACA 0009 aerofoil is used for thickness effects.

Tables-1-3 shows the lift and drag coefficients before and after optimisation. Lift coefficient/induced drag coefficient is seen to increase from a value of 28 to 109 by using the technique brought out in this paper.

Field values of Pressure coefficient for the design case are shown for a particular value of transonic Mach number and angle-of-attack of 20 (Figure-3). The stations F2,F3,F4,F5,F6  are for the vertical spatial displacement.

Estimation of initial optimal pressure difference coefficient ΔCpc and wing warp to produce minimum induced drag at compressible Mach number

Estimation of target pressure difference coefficient ΔCpi for

incompressible flow condition

for minimum induced drag

Inverse design for estimation of Wing warp due to

ΔCp=ΔCpc- ΔCpi

Breaking of warp into camber and twist

Superimposition of thickness

Transonic flow analysis over initial shape and design shape

Figure  1.  Flow chart of the design process

Figure 2. Flow chart for braking optimal warp

Table-1   Values Without Optimisation

| M | CL | Cdi | Cl/Cdi |
| --- | --- | --- | --- |
| 0.1 | .1514 | .0530 | 28.0 |
| 0.8 | .2118 | .0074 | 28.0 |

Table-2 Values after Optimisation at respective  Mach numbers

| M | CL | Cdi | Cl/Cdi |
| --- | --- | --- | --- |
| 0.1 | .1514 | .0017 | 89.0 |
| 0.8 | .2118 | .0027 | 78.5 |

Table-3   Values after Optimisation for target pressure  distribution related to incompressible value of  Mach number= 0.1

| M | CL | Cdi | Cl/Cdi |
| --- | --- | --- | --- |
| 0.8 | .1514 | .0014 | 109 |

| Station | z/c |
| --- | --- |
| F2 | 0.1 |
| F3 | 0.2 |
| F4 | 0.4 |
| F5 | 0.6 |
| F6 | 0.8 |

Figure 3. Design wing profile

Mach number=0.86, =20