# Prediction of Commuting Effects of Varying Hydraulic Cylinder Movements on Soil-Ploughbreast Interaction with a HRP

Lin Zhu, PhD, National Natural Science Foundation of China, China

Min Wei, National Natural Science Foundation of China, China

Feng Luo, National Natural Science Foundation of China, China

Jia-Ru Ge, National Natural Science Foundation of China, China

Tien-Chien Jen, PhD, National Natural Science Foundation of China, South Africa

Abstract

Hydraulic Cylinder (HC) of Horizontally Reversible Plough (HRP) is commonly used for the commuting tillage of HRP. The mechanics of commuting due to HC movements surely affects the tillage performances, but whether this load has adverse effects on the two essential engaging components of HRP, i.e. ploughshare and moldboard is an underlying question. Based on our previously related work, the objective of this study was to characterize the dynamics behaviors of HRP and thus examine the commuting effects on plough-breast and most especially on plough-shank. Uniform motion was applied for HC. A combined finite element analysis (FEA) and multi-body dynamics analysis (MDA) approach was used to characterize the dynamic behaviors of the plough-breast. Five different HC movement scenarios and two actual HRP tilling conditions were involved, and the maximum operation depth of HRP was kept constant at 0.36m. The five HC movement scenarios were specified from 15 to 25mm/s in 2.5 mm increments, and the two actual HRP tilling conditions were HRP commuting either between left and right or between left and middle continuously and alternatively. The loading data due to the HC movements were obtained from an MDA and applied to load a finite element modal of the plough-breast. Results show that the HC movements do result in the maximum stress and strain at the plough-shank, however, has no adverse effects on the service life of the plough-breast. The current HC absolutely favors the actual HRP tillage.

Recent Publications

- Lin Zhu, Chen-Long Yin, Fa Chen. (2006). Application of virtual prototype technology to analyzing the kinetic and dynamic characteristics of the base of the remote cylinder. International Journal of the Japan Society of Mechanical Engineers, Series. C, 41,247~252.

- Lin Zhu, Chen-Long Yin, Fa Chen. (2006). Study on the motion type of the base of the hydro-cylinder based on dynamic simulation software. Machine Tool & Hydraulics, 12, 110~112.

- Lin Zhu, Cheng-Long Yin, Fan-Rang Kong, Li Guo, Xiao-Ming Sun. (2007). Finite element analysis for Plow-breast of Horizontally Reversible Plow. Journal of System Simulation, 19(17):3934~3936.

- Lin Zhu, Shuang-Shuang Peng, Xi-Cheng, Yin-Yin Qi, Wen-Feng Zhang, Liang-Yuan Xu. (2016). Virtual assembly geometric semantics and constraint for an improved three-dimensional model of Horizontally Reversible Plow. International Journal of Mechanical Science and Technology (30): 257~266.

- Lin Zhu, Xi-Cheng, Shuang-Shuang Peng, Yin-Yin Qi, Wen-Feng Zhang, Rui Jiang, Chen-Long Yin. (2016). Three dimensional computational fluid dynamic interaction between soil and plow-breast of horizontally reversal plow. Computers and E lectronics in Agriculture (123): 1~9.

Biography

Lin Zhu is a Professor in the Department of Mechanical Engineering at Anhui Agricultural University (P.R.China). He received his Ph.D. from University of Science and Technology of China in 2008, and was ever a postdoc research associate at University of WisconsinMilwaukee (USA) from 2009. 2. to2010.8. His current research interests include structure and performance optimization of mechanical components, mechanobiology and microfluids.

Email: zl009@mail.ustc.edu.cn    Contact No: +86-13696544372

Notes/Comments: