INFLUENCE OF PHSIO-CHEMICAL PROPERTIES OF SOIL AND PERCOLATING FLUID ON SATURATED HYDRAULIC CONDUCTIVITY OF COMPACTED SOILS IN SELECTED STATES IN NORTH EASTERN NIGERIA

Allen A. Dusa1   and Nachana’a Timothy2

1Department of Agricultural and Bioenvironmental Engineering Federal polytechnic Mubi,

Adamawa State Nigeria

2Department of chemistry Adamawa State University Mubi, Adamawa State Nigeria

Abstract

Hydraulic conductivity is a function of the properties of the fluid flowing as well as the properties of the medium through which it flow. At the moment, conventional laboratory methods using either core or disturbed samples have too many unquantifiable assumptions to make them reliable and reproducible. Field methods on the other hand are too tedious. In this study the influence of physio-chemical properties of soil and percolating fluid on saturated hydraulic conductivity (KS) of compacted soils in selected states in North eastern Nigeria was investigated. Samples were taken from 0 - 0.5 m and 0.5 - 1 m depth from each state. A total of six bulk samples were taken from the field to the laboratory which was dried by spreading in the laboratory for three weeks. The samples were crushed, sieved (using 2mm mesh sieves), compacted in the cylindrical mold using a standard proctor compaction hammer at various densities and saturated by ponding. There after saturated hydraulic conductivity was determined by falling head permeameter. Other soil and percolating fluid properties (depth, particle density, unit weight of water, dynamic viscosity, sodium adsorption ratio and exchangeable sodium percentage and organic matter) were determined and combined using dimensional analysis. KS- model was developed using statistical package for social sciences. The result revealed that KS increases as organic matter increases and decreases as sodium adsorption ratio and texture (clay and silt) increases. The developed KS – model is significant at 5% level which could fairly estimate the saturated hydraulic conductivity of compacted soils at least in the study area.

Keywords:  Soil, Fluid, Properties, Determination, Influence, Compaction and Model