Design of electronic devices for the monitoring of climatic variables in avocado production fields in Colombia

Joaquin Guillermo Ramirez Gil1*; Gabriel Oswaldo Giraldo Martinex2; Morales Osorio Juan Gonzalo3.

1 Universidad Nacional de Colombia, sede Medellin. 2 Physical Engineering, MSc; 3Universidad Nacional de Colombia, Facultad de Ciencias Agrarias, Departamento de Ciencias Agronómicas, Medellín, Colombia. * e-mail: jgramireg@unal.edu.co.

Quantification of climatic variables such as precipitation and temperature and determination of their interactions inside the soil profile is very important in avocado systems as a tool for prompt and appropriate management of several aspects of crop production such as the avocado wilt complex (AWC). Commercial electronic devices are very expensive and present the disadvantage that are made and calibrated for different conditions and usually, most epidemiological models are not suitable for the Colombian conditions. The aim of this work was to design a low cost electronic device for the collection and transmission of climatic variables including moisture and temperature from inside of the soil profile. As a complement, a mobile application was designed based on a different mathematical model that allows to have information in real time, which may be used as an early warning system for the AWC. This electronic device was calibrated for being used in the tropical lower montane humid forest (TLM-hf) and in the tropical lower montane very humid forest (TLM-vhf) life zones in Antioquia, Colombia (sensu Holdridge). Data obtained was correlated with data collected by climatic stations and the quantification within the soil profile of the moisture and temperature by standard methodologies of soil analysis. Besides, the early warning system designed was correlated with data taken in avocado fields associated with AWC. The low cost electronic device showed a correlation higher than 90% compared with data obtained from traditional climatic stations and standard methods of soil analysis. In addition, the early warning system achieved a prediction higher than 80% of the variables associated with AWC.

Biography

Joaquin Guillermo Ramirez Gil is Agricultural Engineer, master in Agricultural Sciences and PhD (c). His work has developed in tropical fruits with emphasis in soil, climate and microorganism interactions. He has published more than 14 articles published in scientific journals.

Presenting author details

Full name: Joaquin Guillermo Ramirez Gil

Contact number: +57 (4) 4309092

Twitter account: not

Linked In account: not

Session name/ number: Agriculture and Environment

Category: oral presentation