# Impact of Dehydration by Successive Pressure Drops on the final characteristics Specialty Coffee

Francy A Vanegas, Ag.E, Universidad Nacional de Colombia, Colombia

Sara V Noel, Ag. E, Universidad Nacional de Colombia, Colombia

Rafael L Sandoval, Ag.E, Universidad Nacional de Colombia, Colombia

Alfonso Parra, M.Sc. PhD, Universidad Nacional de Colombia, Colombia

Sebastian Gutierrez, M.Sc., Universidad Nacional de Colombia, Colombia

### Abstract

The dehydration process reduces the water content of a product to diminish the fragility and to maintain its physical and chemical properties. Due to this, this is a very useful operation in the grain post-harvest, that requires specific conditions for each product. In the case of the specialty coffee beans, as it is delicate, to keep its volatile components, it is not advisable to dry it over 40° C. This limitation made new techniques to develop, to fulfill this requirement and to improve the drying kinetics. The dehydration by successive pressure drops (DPD) technique is slightly new for the heat sensitive products, but it had not been used with the special coffee beans. The intention of this work is to identify the consequences on the operation conditions on the DPD system over the last features of the product. An experimental design was adopted, considering characteristics as the product origin, and the coffee variety. Along the experimental process, a product pre-drying was made to preserve its properties, after that, a DPD was made, with a pressure drop from 74.66 kPa to 5 kPa, in 40° C controlled environment. In conclusion, this process caused less degradation to the grain, an improvement of the organoleptic properties of the product, and a porosity gain of the coffee bean, improving the coffee cup quality. On the other hand, the DPD system operation shortened the drying times, so the dehydration speed increases, in an analogous way the optimization process too, in comparison to the traditional drying systems.

### Recent Publications

- Alonzo M, Montejano G, Allaf K. (2014) Impact of Drying Processes on Strawberry (Fragaria Var. Camarosa) Texture: Identification of Crispy and Crunchy Features by Instrumental Measurement. Journal of texture studies. 45: 246-259.

- Duong T, Al Haddad M, Maache-Rezzoug Z, Allaf K. (2008). Dehydration by Successive Pressure Drops for Drying Paddy Rice Treated by Instant Controlled Pressure Drop. Drying Technology 26(4):443-451

- Kamal I, Gelicus A, Allaf K. (2012) Impact of Instant Controlled Pressure Drop (DIC) Treatment on Drying Kinetic and Caffeine Extraction form Green Coffee Beans. Journal of Food Research 1(1):24-47.

- Pacheco, Sebastian (2016) Diseño, implementación y evaluación de un sistema de deshidratación por cambios cíclicos de presión para residuos agroindustriales. Maestría thesis, Universidad Nacional de Colombia - Sede Bogotá

5. Tellez C, Sobolik V, Montejano G, Abdulla G, Allaf K. (2015) Impact of Swell-Drying Process on Water Activity and Drying Kinetics of Moroccan Pepper (Capsicum annum). Drying Technology. 33:2:131-142.

Biography

Francy Vanegas is agricultural engineering student a senior at University National of Colombia. She has participated in 12th Latin American Congress of Agricultural Engineering with the article Effect of different packages on mango quality (Mangifera Indica L.) Sugar variety and 7th Latin American Congress of Agricultural Engineering Students with the article Behavior of Compression on altered and unaltered sols at different depths (Comportamiento de compression para suelos remoldeados e inalterados a diferentes profundidades de muestreo).

Email: favanegasi@unal.edu.co