# Drying kinetic simulation of a successive pressure drops dehydration (SPDD) machine.

David Andrés Ramirez Gonzalez, Stu, Universidad Nacional de Colombia, Colombia

Yelena Esther Gomez Lara, Stu, Universidad Nacional de Colombia, Colombia

Juan David Alvarez Carpintero, Stu, Universidad Nacional de Colombia, Colombia

Alfonso Parra Coronado, PhD, Universidad Nacional de Colombia, Colombia

Sebastian Gutierrez Pacheco, MSc, Universidad Nacional de Colombia, Colombia

### Abstract

### Drying is one of the most important operations in specialty coffee obtaining because during this process the organoleptic properties of the product are significantly modified, therefore it is necessary to monitor the variables affecting this procedure, mathematical simulation is a feasible alternative in which it is possible to predict the drying kinetic of the coffee bean to determine the operation conditions and correct machine design in order to obtain a high quality product. The drying process was executed by a successive pressure drops dehydration machine, a system that have obtained lower drying times  in comparison to the common drying techniques applied to agricultural products. The objective of this study was to develop a mathematical model able to determine the coffee bean drying time under a successive pressure drops dehydration process (SPDD) at different temperatures.

### The model was developed using 3 drying temperatures (30, 35 and 40 °C), 5 samples of parchment coffee were used in the model construction and 2 for the model validation, in which the moisture content was estimated during the drying process. The model implementation was made using the programming language Microsoft Visual Basic 6.0, the mathematical development was based on the Thompson thin layer drying model, in the model validation the correlation coefficient and the mean squared error were the selected statistical parameters used as adjustment indicators. The simulation goal is to extrapolate the obtained results for different temperatures inside the established range used in the successive pressure drops dehydration process. Moreover the model can be used as a useful tool in the design of thin layer drying machines.

Recent Publications (minimum 5)

- Chua, K.J., Chou, S.K. (2003). On the experimental study of a pressure regulatory system for bioproducts dehydration. Journal of Food Engineering, 62(2): 151–158.

- Haddad, J., Juhel, F., Louka, N., Allaf, K. (2004). A Study of Dehydration of Fish using Successive Pressure Drops (DDS) and Controlled Instantaneous Pressure Drop (DIC). Drying technology, 22(3): 457–478.

- Louka, N. y Allaf, K. (2004). Expansion ratio and color improvement of dried vegetables texturized by a new process “controlled sudden decompression to the vacuum”: application to potatoes, carrots and onions. Journal of Food Engineering, 65: 233 – 243.

- Maache-Rezzoug, Z., Rezzoug, S.A., Allaf, K. (2002). Development of a new drying process: ‘dehydration by cyclical pressure drops’ (D.D.S). Application to the collagen gel, Drying Technology, 20(1): 109–130.

- Parra-Coronado, A., Roa-Mejía, G., Oliveros-Toscón, C.E. (2008). SECAFÉ Parte I: Modelamiento y simulación matemática en el secado mecánico de café pergamino. Revista Brasileira de Engenharia Agricola e Ambiental, 12(4): 415 –427.

Biography

David Andres Ramirez is a last year agricultural engineering student, interested in postharvest, soil management and mathematical modelling, He has participated in 12th Latin American Congress of Agricultural Engineering with the article EFFECT OF USING CASSAVA STARCH IN DIFFERENT CONCENTRATION AS COATING FOR THE LULO (Solanum quitoense) STORAGE also he has participated in 7th Latin American Congress of Agricultural Engineering Students with the article Behavior of Compression on altered and unaltered sols at different depths.

Email: davaramirezgon@unal.edu.co

Notes/Comments: