Optimized Ionic Liquid-based Microwave Assisted Extraction of Piperine 
from Indian Ayurvedic Plant Piper nigrum

Meenu Bhan, Munish Garg, Chanchal Garg*

Department of Pharmaceutical Sciences, Maharshi Dayanand University,

Rohtak, Haryana, India

Abstract

An optimized ionic liquid-based microwave assisted extraction (IL-MAE) method was developed for piperine from Piper nigrum using ionic liquid, 1-butyl-3-methyl-imidazolium chloride ([bmim]Cl) by response surface methodology. Variable microwave-assisted extraction parameters were selected as concentration of [bmim]Cl, extraction time and microwave power while solid-liquid ratio and extraction temperature  were kept constant. Response surface methodology was applied for developing, improving and optimizing the process. A multivariate quadratic regression equation was obtained by central composite design. The 3 D response surface curve and contour plots were obtained by variance of analysis (ANOVA). HPTLC was performed for quantitative and comparative analysis of piperine for IL-MAE with conventional methods. The optimum extraction conditions were 3.0 mol/L, microwave power 700 W, extraction time 9 min, extraction temperature 70º C, solid- liquid ratio 1:10. With this novel IL-MAE method, the yield of piperine is increased from 1.8 % w/w in 7 days (maceration) and 2.03 % in 3 hours (soxhlation) to 4.71 % in 9 minutes (IL-MAE). Meanwhile, there was great reduction in the environmental pollution by using the green solvents/ ionic liquids (ILs), which is a great benefit to all.

Keywords: [bmim]Cl, Piper nigrum, piperine, microwave assisted extraction, response surface methodology.