Title: E-BABE- Simulation of Modified Diffuser Augmented Wind Turbine Performance

Name: Hossam M. Elbakry

Benha University- Shoubra Faculty Of Engineering., CAIRO, Egypt.

Name: Professor: Ahmed A. A. Attia

Benha University- Shoubra Faculty Of Engineering., CAIRO, Egypt.

The objective of this research is to optimize the diffuser configuration of Diffuser Augmented Wind Turbine (DAWT). This study investigates the effect of different diffuser configurations to develop the suitable wind turbine power enhancement. For that purpose, three diffuser configurations have been recommended as an effective design in increasing wind speed in the literature have been used, in addition to a validated model of a small commercial wind turbine (AMPAIR-300). Each configuration is simulated and analyzed using design software Solid-works and Computational Fluid Dynamic (CFD) software Fluent-ANSYS-15, As per the study, The diffuser configuration-1 & 2  have been used to study and validate the effect of adding a diffuser splitter with open angle higher & lower than diffuser open angle at turbine downstream, and with inlet shroud and diffuser flange. The diffuser configuration-3 have been used to study and validate the effect of adding a cone-shape diffuser splitter with open angle mentioned in cone height (which is proportional to diffuser diameter). The simulation of diffuser configuration-1 and diffuser configuration-3 results showed a similar decrement effect in the simulated turbine power coefficient. While for diffuser configuration-2 the diffuser splitter enhanced the simulated turbine power coefficient. Adding inlet shroud directs the wind flow into the inlet of diffuser and the diffuser flange effect on the power enhancement was significant, but, also its induced thrust coefficient was significant. It is recommended to study the optimum dimension of inlet shroud & diffuser flange to minimize the thrust coefficient and to enhance the power coefficient.

Biography

My Name: Hossam M. Elbakry
Age: 34 Years.
Works as A Rotating Equipment Engineer At Oil And Gas Field.
My Experience Is 12 Years Till Now.
I Have Bachelor of Science (Bsc.) at 2007 in Mechanical Engineering (Power Branch).
I Have A Master of Science (Msc.) at 2016, in Mechanical Engineering (Renewable Energy- Wind Energy).
I Am A Doctor Of Philosophy (PhD) Student since 2019, At Benha University- Shoubra Faculty Of Engineering.
I Published A Paper About Renewable Energy.
I Have One Paper Under Review At Renewable Energy Journal (Elsevier).
My Accounts: Hossam M. Elbakry (Linked In) (Elsevier-Mendeley)

Presenting author details 
Full name: hossam mohamed  elbakry
Contact number: 00201001869632	
Twitter account:hossam m. elbakry
Linked In account:hossam m. elbakry

Session name/ number: 
Category: (Oral presentation/ Poster presentation): oral presentation.