Title:Liquid-liquid equilibrium in ternary systems of biodiesel or methyl ester + glycerol + methanol :Experimental data and modeling

Luo Wen1 , Xing Shiyou1, Li Zhibing1,Wang zhiyuan1, Yang Lingmei1 ,

Fu Junying1, Lv Pengmei1

1 Key Laboratory of Renewable Energy, Guangzhou Institute of Energy Conversion, Chinese Academy of Sciences, No. 2, Nengyuan Road, Tianhe Area, Guangzhou 510640, People’s Republic of China

Phase equilibrium information is beneficial for industrial simulation and design for chemical processes. As a complex multi -component system, phase equilibria data of biodiesel systems containing transesterification products are essential for biodiesel production. In this study, liquid-liquid equilibrium ( LLe ) data, saturation curves, and tie-lines were carried out for three ternary systems of palm oil biodiesel/waste oil biodiesel / soybean oil biodiesel + methanol + glycerol at 313.15 K under atmospheric pressure. The results indicated that biodiesel was almost immiscible with glycerol at low methanol content in the system, which was advantageous for the separation of biodiesel from glycerin. As the methanol content rising, mutual solubility of biodiesel and glycerol increased. Saturation curves of these systems exhibited similar characteristics. Liquid liquid Equilibrium (LLE) of methyl oleate / methyl stearate / methyl palmitic + methanol + glycerol were studied at three temperatures (313.15, 323.15, 333.15 K). Mutual solubility of liquids was affected by the temperature. With the rise of temperature , the saturated solubility curve shifted downward, and two-phase region area became narrow, while single-phase region area increased. The experimental data at different temperatures was correlated by NRTL activity coefficient models and binary interaction parameters of the ternary systems were obtained. The calculation results agreed well with the experimental data, indicating that the NRTL model was suitable for these systems．

Biography

Luo Wen has completed her MSc at the age of 26 years from Guangzhou Institute of Energy Conversion(GIEC), Chinese Academy of Sciences, and continues her PHD study in South China Agricultural University. She is an associate professor of GIEC now and has researched on biodiesel production for several years. She has published more than 20 papers in reputed journals. (Up to 100 words)

Presenting author details 
Full name: Luo Wen
Contact number: +8613826077026
Twitter account:
Linked In account:

Session name/ number:
Category: workshop