BIOETHANOL POTENTIAL PRODUCTION TROUGH FERMENTABLE SUGARS SYNTHETIZATED AS OSMOPROTECTANT COMPOUNDS BY PHOTOSYNTHETIC MICROORGANISMS

Dr. Pilar Bremauntz

Mexican Center of Cleaner Production.

National Polytechnic Institute

The production of biofuels is currently regarded as a possible answer in the search for sustainable alternatives for the total or partial substitution of fossil fuels. Bioethanol is one of the most successful biofuels been developed. However, bioethanol production has been limited since it relies on the use of sugar cane or cereals. These materials are important sources of food and their demand, as both a biofuel and a foodstuff has led to a price escalation and may lead to possible shortages. Our group is focused on searching for native microalgae as sources of carbohydrates and bioethanol with the goal of finding a sustainable source of bioethanol. Currently, twelve different strains that reach growth rates between 0.7 - 1.8 g/L and present carbohydrate production under osmotic shock conditions have been isolated. In this work, we present the results obtained with the Chlorella sp. [1] strain, it showed an increment in the sugar production by osmotic stress from 23.64 to 421 mg of sugars/mg of biomass dw, after 24h of osmotic stress with NaCl, 0.4 M..

The Scenedesmus sp. strain showed an increment in the production from 22 to 650 mg/sugar/g of biomass (dry weight), after 24 hours of osmotic shock with 0.1 M NaCl. The osmolytes that were produced after osmotic shock were identified as sucrose and trehalose, both of which are fermentable. These results demonstrate that these strains, through the photosynthetic pathway and osmotic shock, is a potential source of fermentable sugars.