# Impacts of Climate Change on Life Cycle Greenhouse Gas (GHG) Emission Savings of Advanced Bioenergy Systems

Nasir Anka, Garba PhD, Federal University Gusau, Zamfara State, Nigeria

Les John, Duckers PhD, Coventry University, UK

### Abstract

This paper assessed the potential impacts of climate change on life cycle greenhouse (GHG) emission savings of advanced bioenergy systems from corn and soybean. The methodology was underpinned by life cycle thinking. Climate change projections coupled with cropping system modelling (CSM) were linked to life cycle assessment (LCA) modelling. Projected changes in crop yield were calculated using the DSSAT-CSM model with observed climate data for the baseline 1981-1990 period and projected climate change scenarios. The life cycle GHG emission savings was calculated using GaBi 4 LCA software according to the ISO 14044 standard.

Results indicated that under the baseline (1981-1990) scenario, corn integrated bioenergy (CIBE) and soybean integrated bioenergy (SIBE) could save  -10996.7 kg CO2-equiv. ha-1, and -1350.04 kg CO2-equiv ha-1 respectively, of the total life cycle GHG emissions of CO2, CH4, and N2O for the production and utilization of an energetically equivalent amount of fossil-based fuel counterpart, which they displaced. However, model predictions showed that the responses of corn and soybean to simultaneous changes in T, P, and [CO2] were different under different climate change scenarios. In the future period, life cycle GHG emissions savings of CIBE was predicted to decline in all cases ranging from -1.6% to -33.4% compared with the baseline (1981-1990) period. On the other hand, the life cycle GHG emissions savings of SIBE was predicted to increase by +0.1% to +31.6% under some climate change scenarios (e.g., [CO2] = 680; P = +20%; and T = +1.5 oC scenario) and also decline by -0.1% to -82.6% climate change scenarios (e.g., [CO2] = 400; P = -20%; and T = +5 oC scenario).

This revealed that the potential impacts of climate change on energy crops productivity and net life cycle GHG emissions savings could be very large and diverse.

Recent Publications

- Ashiru M. R. and Garba N. A. (2016). Nipa (nypa fruticans) palm as potential feedstock for bioethanol production, International Journal of Scientific & Engineering Research, [Accepted for publication - October 2016].

- N. A. Garba and C. N. Anyanwu (2016).  ‘Laboratory Evaluation of Improved Amfani Charcoal and Firewood Fired Cookstoves’. Nigerian Journal of Solar Energy, 27, pp 44-49.

- Nasir Anka Garba and Umar Zangina (2015). ‘Rice straw and husk as potential sources for mini-grid rural electricity in Nigeria. International Journal of Applied Science and Engineering Research, 4 (6), pp190-197.

- Alsamaq, S., Kingdom, C. and Garba, N.A. (2015). ’Life cycle assessment of agricultural residues utilisation for biogas deployment’.  Weentech Proceedings in Energy, Volume 1: GCED 2015, Eds: Ashish Shukla. World Energy and Environment Technology Ltd.

- Nasir Anka Garba, Les John Duckers, and William John Hall (2014). ‘Climate change impacts on life cycle greenhouse gas (GHG) emissions savings of biomethanol from corn and soybean’.  The International Journal of Life Cycle Assessment, 19 (4), pp 806-813.

- Garba, N. A., Duckers, L. J., and Hall, W. J. (2013). ‘Impact of climate change on first generation biofuels production in the 21st century’. Advances in Manufacturing Technology XXVII, Proceedings of the 11th International Conference on Manufacturing Research. Eds.: Essam Shebab, Peter Ball, and Benny Tjahjono, Cranfield University Press, Cranfield University 2013, pp 549-554.

- Garba, N. A., and Danmallam,  I. M. (2011). ‘Impact of Climatic changes on crops production as it affects biofuels production’. Nigerian Journal of Renewable Energy, 16 (1&2), pp 63-73.

- N. A. Garba and M. A. Rabiu (2011). ‘The development of biodiesel as an alternative fuel: A review’. Nigerian Journal of Renewable Energy, 16 (1&2), pp 33-39.

- Lawal A., Danmallam, I. M., Garba, M. M., and Garba, N. A. (2010). ‘Methods and consequences of improving low temperatures of biodiesel’. Nigerian Journal of Renewable Energy, 15 (1), pp 76-88.

I am currently working as a lecturer in the Department of Chemistry & Biochemistry at Federal University Gusau, Zamfara State, Nigeria. Prior to that I worked (for 7 years) as a Research Fellow with Sokoto Energy Research Centre (SERC) - in the Biomass/Bioenergy Unit, a renewable energy research Centre under the Energy Commission of Nigeria. I hold a Ph.D. degree in Energy, Environment and Sustainability from the Coventry University, UK. I have research interest in bioenergy, sustainability and bioelectrochemistry.

Email: nasiranka@yahoo.com

Notes/Comments: