Tribocorrosion behavior of Ti-10Nb alloy for biomaterial applications

Aline R. Luz, M.D. Universidade Federal do Paraná, Programa de Pós-Graduação em Engenharia e Ciência dos Materiais - PIPE, Brazil.

Carlos M. Lepienski, PhD. Universidade Tecnológica Federal do Paraná, Programa de Pós-graduação em Engenharia Mecânica e de Materiais, Brazil.

Carlos R. Grandini, PhD. Universidade Estadual Paulista, Campus de Bauru, Departamento de Física, Brazil.

Neide K. Kuromoto, PhD. Universidade Federal do Paraná, Programa de Pós-Graduação em Engenharia e Ciência dos Materiais - PIPE, Brazil.

Ti and Ti6Al4V alloy are widely used to replace hard tissues due to their higher biocompatibility, corrosion resistance and suitable mechanical properties. Although, there is a concernment about the long-term release of harmful ions, as aluminum and vanadium by dissolution from the Ti6Al4V alloy. These metals have a poor wear performance, and higher coefficient of friction that restrict to the applications in the biomedical area. Besides, wear debris can result in inflammatory reactions that cause pain and the loss of implants by osteolysis. In order to replace these materials, new beta alloys composed by non-toxic elements, as Nb, Ta, Mo, have been proposed, which are corrosion resistant and have mechanical properties suitable for biomaterials applications. After implantation into bone, implants can be exposed to tribocorrosion conditions, i.e., process combined of corrosion and wear. However, researches about the tribocorrosion behavior of new beta Ti alloys is still very limited. In this study, tribocorrosion properties pure Ti and Ti-10Nb alloy for biomedical applications in phosphate-buffered saline solution were investigated. The test were done by reciprocating tribometer against Al2O3 ball, electrochemical tests and scanning electron microscope images. The results, during the sliding, of open circuit potential demonstrated that the Ti-10Nb alloy was more resistance to corrosion than pure Ti. The coefficient of friction obtained were 0.48 and 0.57 to Ti-10Nb alloy and Ti, respectively. These metals showed a similar wear rate (~0.66. 10-3 mm3/N.m). The images of worn tracks revealed abrasive, adhesive and oxidative wear for all samples. The results disclosed that the coefficient of friction was influenced through microstructure of Ti-10Nb alloy. Whereas, neither mechanics of wear nor wear rate were dependent on the material microstructure. Therefore, in comparing to pure Ti, the addition of beta stabilizer element in the Ti-10Nb alloy improve the corrosion resistance and the coefficient of friction.

Figure 1 - Data from the tribocorrosion test during the sliding period to pure Ti and Ti-10Nb alloy: (a) Open circuit potential and (b) Coefficient of friction. Test performed in PBS solution, with 1 N, 1 cm / s, 1250 cycles, 40 Hz, Al2O3 ball (Ø= 6 mm).

Acknowledgements

The authors would like to thank the Centro de Microscopia da UFPR, and the LaBES – Laboratório de Biomateriais e Engenharia de Superfícies PUC-PR for the facilities, CNPQ and CAPES for financial support.

Recent Publications

- A.R. Luz, G.B. De Souza, C.M. Lepienski, C.J.M. Siqueira, N.K. Kuromoto, Tribological properties of nanotubes grown on Ti-35Nb alloy by anodization, (2018).

- S. Ehtemam-Haghighi, K.G. Prashanth, H. Attar, A.K. Chaubey, G.H. Cao, L.C. Zhang, Evaluation of    mechanical and wear properties of Ti-xNb-7Fe alloys designed for biomedical applications, Mater. Des. 111 (2016) 592–599.

- J.M. Cordeiro, V.A.R. Barão, Is there scientific evidence favoring the substitution of commercially pure titanium with titanium alloys for the manufacture of dental implants? Mater. Sci. Eng. C 71 (2017) 1201–1215.

- Y.S. Lee, M. Niinomi, M. Nakai, K. Narita, K. Cho, Predominant factor determining wear properties of β-type and ( α+β )-type titanium alloys in metal-β α β to-metal contact for biomedical applications, J. Mech. Behav. Biomed. Mater. 41 (2015) 208–220.

- M.A. Hussein, A.S. Mohammed, N. Al-Aqeeli, Wear characteristics of metallic biomaterials: a review, Materials (Basel) 8 (2015).

Biography

Aline R. Luz is an experimental physics professor. In February 2019, she will finish her PhD in engineering and materials sciences from the University Federal of Paraná. In 2016, she won Trainee Award of 10th World Biomaterials Congress, once her work was one of the best of congress. The focus of her PhD thesis was the tribocorrosion and mechanical properties of nanotube arrays grown on titanium and on new titanium alloys. Her research domain, in engineering and materials sciences, is concentrated in characterization of new beta titanium alloys and films obtained on metals surface by anodic oxidation technique. The aim of investigations new beta titanium alloys is propose new materials to replace the hard tissue with better properties than commercial materials, as titanium and Ti-6Al-4V alloy. While the surface treatment is request to turn the metal surface bioactive, so, it improve the biocompatibility of implant.

email: arossettoluz@gmail.com

Notes/Comments: