Improving the Efficiency of Mg-Co Nano-ferrites for Adsorption of Crude Oil from Aqueous Solution by Blending them with Chitosan Hydrogel

Zizi I. Abdeena , Amina I. Ghoneimb,*

a Petrochemicals Department, Egyptian Petroleum Research Institute, Nasr City, Cairo, Egypt.

b Physics Department, Faculty of Science, Tanta University, Tanta, Egypt.

Abstract

The efficiency of the as-prepared Mg0.25Co0.75Fe2O4 spinel nanoparticles for adsorption of crude oil from aqueous solution was improved by blending them with hydrogel (CH) prepared using epichlorohydrin as cross-linker. Mg0.25Co0.75Fe2O4 nanoparticles was prepared by the chemical co-precipitation method and characterized by using XRD, FT-IR, SEM and TEM. DTA/TGA curves showed that; the net weight loss of the samples heated from room temperature up to 1000 ºC lies in the range 2.2– 26.5 % weight, where the maximum weight loss appeared at 100 ºC and 614 ºC. The prepared blending (CH/Mg0.25Co0.75Fe2O4) and Mg0.25Co0.75Fe2O4 were characterized by FT-IR and SEM micrographs. The effect of the Mg0.25Co0.75Fe2O4 ratio on the water uptake of CH/Mg0.25Co0.75Fe2O4 was studied and optimized.