The effect of Ga3+  and Cu2+  co-doping on  ionic conductivity of Ceria Ceramics as Solid Electrolyte for IT-SOFCs

Monika Singh*, Akhilesh Kumar Singh**

School of Materials Science and Technology, Indian Institute of Technology (Banaras Hindu University), Varanasi-221005, India

*E-mail: monikas.rs.mst15@itbhu.ac.in  & aksingh.mst@iitbhu.ac.in

### Abstract

The current investigation draw an attention to the effect of Ga3+ and Cu2+ addition on the structure, surface morphology and oxygen-ionic conductivity of ceria ceramics in the composition Ce0.8Ga0.2-xCuxO2-  for the promising candidate as the solid electrolyte material in the solid oxide fuel cells working in intermediate temperature range (IT-SOFCs). Ultrafine Ce0.8Ga0.2-xCuxO2- (for 0x0.2) nanoceramics were prepared via glycine nitrate auto-combustion method. Phase identification, microstructure, and ionic conductivity of all the ceria ceramics were observed by powder XRD, SEM, TEM, and impedance analyses measurement were used to analyze phase identification, microstructure, and ionic conductivity of all the ceria ceramics respectively. Similar to cerium oxide cubic fluorite type structure having Fm-3m space group was confirmed by powder XRD followed by Rietveld structural analysis for all the co-doped systems. The density of all samples was found above 85% after sintering at 1300ºC for 4 hrs. The presence of oxygen vacancies in all the compositions were revealed by Raman spectra. Thermal analysis for change in weight was carried out by TGA. Thermal expansion coefficient of the developed electrolytes matches with the commonly used electrode materials. The optimum composition Ce0.8Ga0.05Cu0.15O1.825 was found to reveal the maximum ionic conductivity with least activation energy among all the existing co-doped ceria ceramics. These characteristics compose it a potential applications in the IT-SOFC as the electrolyte material.

### Images (If any)

Recent Publications (If any)

- Uday Pratap Azad, Monika Singh, Sourav Ghosh, Ashish Kumar Singh,Vellaichamy Ganesan, Akhilesh Kumar Singh, Rajiv Prakash.” Facile synthesis of BSCF perovskite oxide as an efficient bifunctional oxygen electrocatalyst” .International Journal of Hydrogen Energy 43(45) DOI: 10.1016/j.ijhydene.2018.09.134

- Monika Singh, Akhilesh Kumar Singh “Studies on structural, morphological, and electrical properties of Ga3+ and Cu2+ co-doped ceria ceramics as solid electrolyte for IT-SOFCs”.International Journal of Hydrogen Energy DOI: 10.1016/j.ijhydene.2019.09.084

- Monika Singh, Dinesh Kumar, Akhilesh Kumar Singh, “Synthesis and structural study of cerium substituted La0.4Ca0.6MnO3 solid oxide fuel cell electrode material”, AIP conference proceedings.

- Dinesh Kumar, Monika Singh, Akhilesh Kumar Singh, “Crystallite size effect on  lattice strain and crystal structure of Ba0.25Sr0.75MnO3 layered perovskite mangnite”, AIP conference proceedings.

- Vinita, Madhu Tiwari, Neha Agnihotri, Monika Singh, Akhilesh Kumar Singh, and Rajiv Prakash “Nanonetwork of Coordination Polymer AHMT-Ag for the Effective and Broad Spectrum Detection of 6-Mercaptopurine in Urine and Blood Serum” ACSOmega DOI: 10.1021/acsomega.9b01122

- Monika Singh, Pragyanand Prajapati, Akhilesh Kumar Singh” Developing a reduction resistant layer with SrNi0.8Mo0.2O3-x mixed–oxide for Sm0.2Ce0.8O2-x based Solid Oxide Fuel Cells” AIP conference proceedings DOI: 10.1063/1.5113461

- Monika Singh, Akhilesh Kumar Singh “Effect of Alkali Carbonates Addition on tri-doped ceria: Structure, Microstructure, Ionic conductivity and Charge transport properties for LT-SOFCs Applications.(Submitted)

- Monika Singh, Vijayeta pal, Akhilesh Kumar Singh Investigation of Dielectric Relaxation and ion Dynamics of Ce0.76Pr0.08Sm0.08Gd0.08O2- nanocrystalline Solid Electrolyte ion conductor material today proceedings

Biography

Monika Singh is  5 th year  Ph.D. student in School of Materials science and technology ,Indian Institute of Technology, Banaras Hindu University(IIT-BHU) She is pursuing her research work  under the supervision of Dr. Akhilesh Kumar Singh (Associate professor). She received his MSc in Physics from the Banaras Hindu University. She has expertise in the study of flash sintering behaviors and its applications. Her research is focused on Study of ceramic and composite based Electrode and electrolyte materials for Intermediate Temperature Solid Oxide Fuel Cells (IT-SOFCs).  She found the effect of different dopants addition on structure, morphology, sintering, electrical properties of Ceria ceramics as electrolyte for IT-SOFCs. She also successfully improved the ionic conductivity of ceramic oxides as solid electrolytes by addition of carbonates. She has published research papers in the international reputated journals and international conference proceedings.

Email: monikas.rs.mst15@itbhu.ac.in

Contact no : 9151577458