# Studies on removal of Cu2+ by Cetyl Pyridinium Bromide (CPB) modified red clay from Maharashtra

R.W.Gaikwad, A.R.Warade and S.L.Bhagat,

Pravara Rural Engineering College, Loni,

Dist: Ahmednagar (MS)-413736, India

### Abstract (300 word limit)

Statement of the Problem: Acid Mine Drainage (AMD) is a serious hazard to human health, animals and ecological systems. This is because AMD contains heavy metal contaminants, such as Cu2+, Fe3+, Mn2+, Zn2+, Cd2+ and Pb2+ which is not biodegradable and thus tends to accumulate in living organisms, causing various diseases and disorders. Unmodified red clay and CPB modified red clay with various loadings of cetyl pyridinium bromide (CPB) have been used as adsorbents to remove Cu2+ from aqueous solution. CPB-RC has presented higher Cu2+ adsorption more efficiency than unmodified red clay. The purpose this study was to investigate the equilibrium as well as the rate of adsorption of Cu2+ in aqueous solution and efficiency of clay based adsorbent prepared using red clay fro Maharashtra with its different conditions i.e. unmodified red clay (URC) and CPB modified red clay (CPB-RC).

Methodology & Theoretical Orientation: The adsorption of Cu2+ onto CPB-modified red clay as a function of contact time, initial Cu2+ concentrations, adsorbent dose, pH and temperature was investigated. The isotherm experiments were carried out with fixing the adsorbent dose and varying the initial concentration of Cu2+. The results indicated that CPB-modified red clay showed greatly 
enhanced adsorption capacity. Findings revealed that the Langmuir 
isotherm best described the Cu2+ adsorption on the adsorbent. The 
adsorption isotherm was well fitted to the Langmuir model.

Conclusion & Significance: The findings of this study showed that the modification of red clay can achieve higher adsorption capacity for heavy metal removal. In addition, based on the results, red clay being modified with CPB as a modifier had good efficiency for the adsorption of Cu2+.

### Image

Recent Publications (minimum 5)

- J.H. Potgieter  et.al.   (2006) Heavy metals removal from solution by palygorskite clay, Minerals Engineering ,Volume 19, Issue 5, PN. 463–470.

- Heba H. El-Maghrabi and  Sara Mikhail (2014)  Removal of Heavy Metals via Adsorption using Natural Clay Material,  Journal of Environment and Earth Science, Vol.4, No.19, PN38-46.

- H. A. Talaat et.al. (2011) Evaluation of Heavy Metals Removal Using Some Egyptian Clays, 2011 2nd International Conference on Environmental Science and Technology IPCBEE vol.6 , IACSIT Press, Singapore,PN.37-42.

- Prakash N. et.al. (2013)  Influence of clay on the adsorption of heavy metals like copper and cadmium on chitosan , Environ Sci Pollut Res Int., Feb;20(2):,PN. 925-938.

- Mohammad W. Amer , Fawwaz I. Khalili  and Akl M. Awwad (2010) Adsorption of lead, zinc and cadmium ions on polyphosphate-modified kaolinite clay, Journal of Environmental Chemistry and Ecotoxicology Vol. 2(1) PN. 001-008.

Biography (150 word limit)

Dr. R.W.Gaikwad has his expertise in removal of heavy metals from wastewaters. His research has focused on modeling, optimization and removal of heavy metals from wastewater, with emphasis in Environmental Technology. The emphasis is both on the fundamental understanding and modeling of the key heavy metal removal that take place in environmental systems, and on the development of improved technologies for the treatment and management of liquid and solid wastes. Dr. Gaikwad has directed or co-directed research and training projects. He has had the opportunity to lead an extensive research effort involving field data collection and modeling of the chemical engineering system. He has serving as an independent consulting engineer. He has designed and conducted numerous short courses and special training programs in process modeling & simulation and Environmental Engineering. He is a Member of American Institute of Chemical Engineers, Indian Institute of Chemical Engineers, Institution of Engineers (India).

Email:    rwgaikwad@yahoo.com

Notes/Comments: