# Title: Simulation studies on finishing reactive distillation column

Raj Kumar Gupta, PhD, Thapar Institute of Engineering and Technology, Patiala, India

Damandeep Singh, PhD, IOLCP, Barnala, India, Vineet Kumar, PhD, IIT (ISM) Dhanbad, India

### Abstract (300 word limit)

Statement of the Problem: Reactive distillation (RD) process combines two operations (reaction and distillation) in a single vessel. RD may offer many advantages over the individual reaction and distillation processes where the two operations have similar operating temperature and pressure.  In case of a finishing RD column, most of the reaction (near equilibrium) takes place in the pre-reactor and the reacted mixture along with the catalyst is fed to the distillation column where the reaction can move in the forward direction due to simultaneous product withdrawal. The existing plants based on conventional process (distillation followed by reaction) can be revamped to obtain the conversion beyond equilibrium limitation.

In this study, a finishing RD column for the ethyl acetate (EtAc) is simulated using rate-based model of Aspen Plus. The results are compared with the data collected from a plant scale revamped RD column for ethyl acetate production. The rate-based model results matched the column profiles (plant data) better than the equilibrium (EQ) model results. The EQ model predicted higher EtAc generation rates, than the rate-based model, as it ignored mass transfer resistance. The results showed that the product purity increased with reflux rate. However, the overall conversion of ethanol in the column decreased with increased reflux.

Figure 1: Ethyl acetate composition profile

Recent Publications (minimum 5)

- Thakur R, Barman, S, Gupta RK (2017) Kinetic investigation in Transalkylation of 1,2,4 Trimethylbenzene with toluene over rare earth metal modified large pore zeolite. Chemical Engineering Communications 204: 254-264.

- Thakur R, Gupta RK, Barman S (2017) A comprehensive study of catalytic performance of rare earth metal modified beta zeolites for synthesis of cymene. Chemical Papers 71: 137-148.

- Kaur H, Bulasara, VK, Gupta RK (2016) Effect of carbonates composition on the permeation characteristics of low-cost ceramic membrane supports. Industrial and Engineering Chemistry 44: 185-194.

- Singh D, Gupta RK, and Kumar V (2015) Simulation of a plant scale reactive distillation column for esterification of acetic acid. Computers and Chemical Engineering 73: 70-81.

5. 	Purandare PS, Lele M, Gupta RK (2015) Investigation on thermal analysis of conical coil heat exchanger. International Journal of Heat and Mass Transfer 90: 1188-1196.

Biography

Raj Kumar Gupta is Professor of Chemical Engineering, at Thapar Institute of Engineering and Technology Patiala. He joined the Department of Chemical Engineering in 1999 and is currently Head of the Department. His research interests are in the areas of modeling and simulation of chemical processes, specifically, FCC modeling and simulation, Reactive distillation modeling and simulation, Multiphase reaction kinetics, RTD modeling, CO2 adsorption, Heat transfer enhancement, Water and wastewater treatment.

Email:rkgupta@thapar.edu

- Full Name: Dr. Raj Kumar Gupta

- Personal Email: rkgupta@thapar.edu

- Mobile Number: +91-9872206814

- Linked In:

- Facebook:

- Session Name/ number:

- Category: (Oral presentation/ Poster presentation): Oral presentation

- Date of Birth: August 07, 1970

- Postal Address: Professor, Department of Chemical Engineering, TIET, Patiala-147004, India