Title: A novel and fast approach to monitor cell disruption efficiency

Name: Britta Eggenreich1, Vignesh Rajamanickam1,2, Christoph Herwig1,2 and Oliver Spadiut1,2

1Research Division Biochemical Engineering, Institute of Chemical Engineering, Vienna University of Technology, Vienna, Austria

2 Christian Doppler Laboratory for Mechanistic and Physiological Methods for Improved Bioprocesses, Institute of Chemical Engineering, Vienna University of Technology, Vienna, Austria

The bacterium Escherichia coli is a well-studied recombinant host organism with a plethora of applications in biotechnology. High valuable biopharmaceuticals, such as recombinant enzymes, antibody fragments and growth factors, are currently being produced in E. coli. These molecules are usually produced intracellularly which is why cell disruption is required as the first step in the downstream process. For that purpose high pressure homogenization is the system of choice since it is scalable and can be run in continuous mode. However, it is crucial to determine cell disruption efficiency to 1) avoid product loss in intact cells, but also to 2) avoid unnecessary long disruption cycles and thus harm the product. Usually, cell disruption efficiency is evaluated either by determination of colony foming units (CFUs) or photometric measurements of nucleic acids and protein content in the lysate. However, these methods are both characterized by disadvantages, as CFUs can only be counted on the next day, resulting in great time delay, and photometric measurements are affected by matrix effects.

In this study, we implemented a novel online tool based on UV chromatogram fingerprints and chemometric techniques to monitor cell disruption efficiency. We used 1) measurement of the total protein content in the supernatant, 2) determination of CFUs and 3) flow cytometry as reference analytics to validate this novel tool. Finally, we performed a Design of Experiments study, where we changed the factors “concentration of biomass per ml buffer”, “number of homogenization cycles” and “pressure during homogenization” to analyze and optimize the Unit Operation high pressure homogenization for a recombinant E. coli strain producing a highly valuable antibody fragment. Summarizing, we could nicely demonstrate the power of the novel online tool, which will certainly facilitate the evaluation of this crucial Unit Operation in the future.

Keywords: Escherichia coli, high pressure homogenization, online monitoring, chromatogram fingerprinting

Presenting author details 
Full name: Britta Eggenreich
Contact number: +43(1)58801 – 166496 
Twitter account: -n.a-
Linked In account: -n.a-

Session name/ number: -n.a-
Category: Oral Presentation