# Quantitative Determination of Humic Substances in a Wide Range of Calibration by Flow Injection Systems. Part Two: The Simultaneously Analyze of Changes between the Flow Rate and the Temperature by Chemometric Approaches

İsmail Tarhan, Selçuk University, Faculty of Science, Department of Biochemistry, TURKEY

Hüseyin Kara, Selçuk University, Faculty of Science, Department of Chemistry, TURKEY

### Abstract

### In this present study, the optimization of the flow rate and the temperature of humic substances in the quantitative determination by flow injection systems was carried out within the framework of chemometric approaches. The optimal points were determined by following values of the response from the parameters owing to various mathematical equations. For this purpose, the design 5-level consisting the flow rate values (0.50, 0.75, 1.00, 1.50 and 2.00 mL/min respectively), and the temperature values (20, 25, 30, 35, 40 oC respectively) was used. The standards of the humic acids (0, 1, 5, 10, 25, 50, 125, 250, 350, 500, 1000, 1250, 1500, 1750 and 2000 mg/L) have been injected into the system at least three times according to the above experimental design. The reproducible of the obtained results was investigated by accepting response value their ratio height/width of the peaks obtained from the analysis (Figure 1).

### As a result, quantitative determination of humic substances by flow injection systems is very useful, economic, quick and accurate when it is compared with other standard methods.

### Figure 1. The 3D profiles of detection responses affected by the flow injection analysis parameters; X1 flow rate of mobile phase and X3 the system temperature.

Recent Publications

- Ismail Tarhan and Huseyin Kara (2016) “A Chemometric Study: Automated Flow Injection Analysis Method for the Quantitative Determination of Humic Acid in Ilgın Lignite”, Arabian Journal of Chemistry 9:713-720.

- Fakhar N. Memon, H. Filiz Ayyildiz, Huseyin Kara, Shahabuddin Memon, Adnan Kenar, Muhammad Khan Leghari, Mustafa Topkafa, Syed Tufail Hussain Sherazi, Naeem Ahmed Memon, Fatih Durmaz, Ismail Tarhan (2015) Application of central composite design for the optimization of on-line solid phase extraction of Cu2+ by calix[4]arene bonded silica resin. Chemometrics and Intelligent Laboratory Systems 146: 158-168.

- Ismail Tarhan, H. Filiz Ayyıldız, Mustafa Topkafa, Fatmanur Arslan, Aslan Tas, S. T. Hussain Sherazi, Huseyin Kara (2015) Chemical and Spectroscopic Characterization of Humic Acid Isolated from Ilgın Lignite, Turkey. International Journal of Scientific and Technological Research 1(1): 176-183.

- Fatma Nur Arslan, Huseyin Kara, Hamide Filiz Ayyildiz, Mustafa Topkafa, Ismail Tarhan, Adnan Kenar (2013) A Chemometric Approach to Assess the Frying Stability of Cottonseed Oil Blends During Deep-Frying Process: I. Polar and Polymeric Compound Analyses. J. Am. Oil Chem. Soc. 90:1179-1193.

- H. Filiz Ayyildiz, Mustafa Topkafa, Fatma Nur Arslan, Fatih Durmaz, Semahat Kucukkolbasi, Ismail Tarhan, Huseyin Kara (2012) Removal and Preconcentration of Cobalt Ions from Aqueous Media Using ImHA Packed Column by On-line SPE System. Water air soil pollution 223(7):3817-3830.

Biography

İsmail Tarhan has her expertise in flow injection analysis and chromatography. He works in Biochemistry Department since 2010. Mainly fields are flow injection, humic substances, monolithic columns, infrared analysis, and chemometric designs.

Email: ismtarhan@gmail.com