Tobacco – A Platform for Efficient Biofuel Production: Pretreatment to Bioethanol production from  Lignocellulosic Biomass of Tobacco.

Kokila S, PhD, Department of Biochemistry, Centre of Research and Post Graduate Studies, Indian Academy Degree College, Bangalore, Karnataka, India.

Mahuya De Ghosh, PhD, Department of Chemistry, Centre of Research and Post Graduate Studies, Indian Academy Degree College, Bangalore, Karnataka, India.

Prashanthi Karyala, PhD, Department of Biochemistry, Centre of Research and Post Graduate Studies, Indian Academy Degree College, Bangalore, Karnataka, India.

Cotek Temitayo, Inla Sravani, Department of Biochemistry, Centre of Research and Post Graduate Studies, Indian Academy Degree College, Bangalore, Karnataka, India.

Abstract

Statement of the Problem:

The escalating industrial and domestic demands on non-renewable energy resources have led to the rapid depletion of fossil fuels. This has resulted in the emergence of bioethanol derived from fermentation of food crops such as maize and corn which has increased the prices of food commodities1. Second generation bioethanol based on raw materials rich in complex carbohydrates such as cellulose reduces the competition with the food industry2. Tobacco is grown in large fields all over the world and generates multiple harvests per year, thus producing large amounts of inexpensive green biomass. The process to obtain second generation bioethanol involves four basic steps:  pretreatment, enzymatic hydrolysis, sugar fermentation, and ethanol recovery3.

Methodology & Theoretical Orientation:

The dried tobacco leaves and stalk were pretreated with water, buffer (0.1M Citrate buffer) and dilute acids (H2SO4, HCl, HNO3 at 1%, 4% and 6%) at different temperatures (60◦ C,  autoclave - 121◦ C and 130◦ C) and microwave treatment (700 W, 2 min)4. The percentage of cellulose in the pre and post treated biomass was estimated by the method of Updegraff5. The pretreated biomass was subjected to enzymatic hydrolysis using cellulase from Trichoderma reesei (~700 U/g of substrate) and β-glucosidase (60 U/g of substrate). The total yield of glucose and ethanol produced for each pretreated biomass was assayed by standard procedures6,7.

Findings:

A considerable loss of biomass was observed after pretreatment with dilute acids compared to pretreatment with steam in water or citrate buffer. The highest glucose and ethanol yield was obtained in the pre-treated stalk with steam at 121◦ C in citrate buffer.

Conclusion & Significance:

Results from the presented experimental work indicate that leaves and stalk of tobacco have a vast potential for the production of sugars that eventually can be used for producing bio-ethanol. Despite declining cigarette sales worldwide, the use of tobacco to produce bio-ethanol can be an alternative approach to save tobacco farmers. As tobacco is not a food source it will not drive up food prices.

Recent Publications:

- Smith AM. Prospects for increasing starch and sucrose yields for bioethanol production. Plant J. 2008 May;54(4):546-58.

- Hood EE. Plant-based biofuels. F1000Res. 2016 Feb 17;5.

- Bensah EC and Mensah M. Chemical Pretreatment Methods for the Production of Cellulosic Ethanol: Technologies and Innovations. Int J Chem Engg. Vol. 2013, Article ID 719607, 2013. doi:10.1155/2013/719607.

- Li A, Antizar-Ladislao B, Khraisheh M. Bioconversion of municipal solid waste to glucose for bio-ethanol production. Bioprocess Biosyst Eng. 2007 May;30(3):189-96.

- Updegraff DM. Semimicro determination of cellulose in biological materials. Anal Biochem. 1969 Dec;32(3):420-4.

- Luchsinger WW, Cornesky RA. Reducing power by the dinitrosalicylic acid method. Anal Biochem. 1962 Oct;4:346-7.

- Sayyad SF, Chaudhari SR, Panda BP. Quantitative determination of ethanol in arishta by using UV-visible spectrophotometer. Pharm and Biol Evaluation. October 2015; vol. 2 (Issue 5): 204-207.

Biography

Karyala Prashanthi has more than 16 years experience in teaching and research in Host – pathogen interactions of very diverse pathogens such as Rotavirus, Mycobacterium and Dengue virus. She has experience in virology, molecular biology and computational biology that include design principles of biological networks, computational modeling and simulation of biological systems and networks, systems biology and metabolic network analysis.

Karyala Prashanthi

Contact No. 91-9900130316

Email id: karyalaprashanthi@gmail.com

prashanthi@indianacademy.edu.in

Mahuya De Ghosh has more than 19 years experience in teaching and research in natural product and carbohydrate chemistry. Her research areas are organic synthesis and characterisation, green chemistry and polymer chemistry.

Mahuya De Ghosh

Contact No. 91-9986408743

Email id: demahuya@yahoo.com

mahuyadeghosh@indianacademy.edu.in