# Kinetic and thermodynamic studies of polyvinyl chloride composite of manganese oxide nanosheets for the efficient removal of dye from water

Tooba Saeed, M.Phil. NCEPC, University of Peshawar, Pakistan

Abdul Naeem, Ph.D., NCEPC, University of Peshawar, Pakistan

### Abstract

### Manganese oxide nanosheets and manganese oxide composite with polyvinyl chloride (MnO2-PVC) were synthesized by the oxidation method for the efficient removal of congo red (CR). The Fourier transform infrared spectroscopy, X-ray diffraction, energy dispersive X-ray, surface area, point of zero charge and thermogravimetric analysis were performed to verify the newly synthesized adsorbents. After functionalizing the surface of MnO2 nanosheets with PVC, the PZC and SA were amplified from 4.10 and 214 m2 g1 to 5.01 and 226 m2 g1 respectively. The batch adsorption results showed that the removal capacity of CR on both the adsorbents decreased with the increase of pH and time, but increased with the increase of adsorbent dosage. However, due to the high stability, porosity and greater surface area, the PVC composite of MnO 2 was found to exhibit 15 times greater CR removal efficiency than its parent MnO 2 . Furthermore, the thermodynamic parameters specified that CR adsorption onto both the adsorbents was exothermic, spontaneous and film diffusion accompanied by the intraparticle diffusion is the rate controlling step. These results validate that MnO 2 composite with PVC is a useful, eco-friendly, competent candidate for dye removal from wastewater.

Recent Publications

- Tooba Saeed, Abdul Naeem et al., Synthesis of Chitosan Composite of Metal-Organic Framework for the Adsorption of Dyes; Kinetic and Thermodynamic Approach, Journal of Hazardous Material, 2021, in press.

- Afsar Khan, Abdul Naeem, Tahira Mahmood,  Zahoor Ahmad, Muhammad Farooq, Tooba Saeed, Mechanistic study on methyl orange and congo red adsorption onto polyvinyl pyrrolidone modified magnesium oxide, International Journal of Environmental Science and Technology, 2021 1, 1112.

- Israf Ud Dina, Tooba Saeed, Zahoor Ahmad, Abdul Naeem, A Comparative Study for the Effect of Calcination on the Temperature-Dependant Magnetic Properties of Cobalt Ferrite Nanoparticles Journal of Superhard Materials, 2021, 43,  4, 278–284.

- Tooba Saeed, Abdul Naeem, Tahira Mahmood, Kinetic and thermodynamic studies of polyvinyl chloride composite of manganese oxide nanosheets for the efficient removal of dye from water Water Science & Technology, 2021, 1, 2–16.

- Israf Ud Din, Tooba Saeed, Zahoor Ahmad, Abdulrahman I. Alharthi, Mshari A. Alotaibi and Abdul Naeem, Reporting the magnetic profile of cobalt ferrite nanoparticles at different temperatures, journal International Journal of Materials Research, 2021, 33, 1–14.

Presenting author details 
Full name: Tooba Saeed
Contact number: 03329291972
Twitter account:
Linked In account:

Session name/ number: 
Category: Oral presentation