Evaluate the internal structure of six fiber post and stress distribution analysis.

ABSTRACT

The composite fiber / resin, among which the fiber post are demonstrate tensile strength, so that the type of fiber is important, its distribution, fiber matrix relationship going determine the structural quality. That is why the finite element method (FEA), we can model the system prosthetically restored tooth so it could be related to its internal structure with mechanical testing. Objective: To evaluate the internal structure of different fiber posts and correlate the distribution of stress in a central incisor restored with fiber post. Materials and Methods: We evaluated cross sections in 6 types of glass fiber posts (RTD ® lock Macro, FRC Postec Plus Ivoclar Vivadent ®, Coltene Whaledent ParaPost White Fiber ®, Coltene Whaledent ParaPost Lux ® Fiber, Glass Fiber Post Superpost ® and Rx Reforpost Angelus ®). Scanning electron microscope images were obtained at 100μm, 500μm and 2500μm increase. Calculated: total area, defect area, uniformity in fiber diameter and fiber-matrix connection. Central incisor was modeled with Soliword. The (FEA) was performed by the software ANSY. The variables included in the modeling were the modulus of elasticity and Poisson's ratio for all components of the model. Was applied a load of 200 N to calculate the distribution of stress on the structure. Results and conclusions: The most homogeneous, compact and no imperfections introduced were the FRC Postec Plus and Macro lock. The White Fiber ParaPost showed fiber diameters more homogeneous and less concentration of fibers. The Rx Reforpost had higher number of obvious defects. The fiberglass have different qualities structural defects and some show low consistency in its fibers. For FEA was observed that Macro Lock was the best stress distribution obtained. This is due to having elastic modulus which is similar to that of dentin and is directly proportional to their structural quality. Was observed for each group behaved differently depending on the material of the fiber. A higher modulus was observed an increase in the distribution of stresses therein, decreasing the stress in the dentin. The structural quality is an important aspect to consider when selecting a bolt that requires a suitable mechanical performance in reconstructions of anterior stump.

Keywords: fiber posts, internal structure, matrix, fiber, internal defects, finite element analysis.