## Measurement and analyses of concentrations of methane (CH4) and nitrogen-dioxide (NO2) gases at a quarry site.

Orji Prince1,2, Obiegbuna Dominic1, Anoruo Chukwuma1, Chiemeka Ikechukwu2

1University of Nigeria, Nsukka, Nigeria.

2Abia State University, Uturu, Nigeria

Measurement and analyses of the concentrations of NO2 and CH4 gases in ambient air at Setraco quarry site, Abia State, Nigeria (latitude 05⁰33N and 06⁰03N, and longitude 10⁰E and 07⁰29E) were carried out in this study. The gases were measured using two automatic air samplers namely EM-4 Type Multi-P detector and AIKE EM-201 Gas meter. The measurements were made at three sampling points; the engine house, petrol station and crushing site for 12 weeks (7th August – 6th November, 2017) at one (1) hour interval per session for three sessions (morning, afternoon and evening) in a day. It was observed that at petrol station, the mean concentration of methane was highest with a value of 3.14±0.8% which is much higher than World Health Organization (WHO) recommendation of 0.1% exposure limit for 8hours working time. The lowest mean concentration at the petrol station was 0.39±0.08% which is still quite high and above deleterious level or the WHO acceptable limit. At the Engine House, the highest mean concentration of NO2 measured was 375±4.0ppm, which is higher than World Health Organization (WHO) and Federal Environmental Protection Agency (FEPA) acceptable limit of 4.89ppm for weekly mean concentration. The Air quality index (AQI) calculation for the NO2 concentration of 375±4.0ppm yielded a health-threatening value of 153 especially for people with heart and breathing problem. These results imply that NO2 is emitted more at the Engine house, less at the Petrol station and much less at the Crushing site while CH4 is emitted more at the Petrol station than Engine house and Crushing site.

Biography

Orji Prince Orji has a Masters Degree in Atmospheric Physics from Abia State University, Uturu, Nigeria. He is currently undergoing a PhD programme (with a research interest in “dampening of signal strength for propagation due to atmospheric constituents)  in same institution. Orji is currently an academic staff of the University of Nigeria. He has published five (5) journal papers with reputable journals.

Full name: Orji, Prince Orji

Contact number: +2348138513447

Twitter account: Nil

LinkedIn account: Prince Orji

Session name/number: Track 4 – Pollution Control

Category: Oral Presentation

## THE EARTH’S LOWER ATMOSPHERE

The earth’s atmosphere is a thin, gaseous envelop comprised mostly of Nitrogen (N2) and Oxygen (O2), with small amount of other gases, such as water vapour (H2O), carbon dioxide (CO2) and trace gases such as methane (CH4) which is the most abundant trace gas in the atmosphere. The present day atmosphere is quite different from the natural atmosphere that existed before the industrial revolution in terms of chemical composition. If the natural atmosphere is considered to be “clean” then this means that clean air cannot be found anywhere in today’s atmosphere (Sara, 2014).

Defining “air pollution” is not simple. One could claim that air pollution started when human began burning fuels. In other words, all man-made (anthropogenic) emissions into the air can be called air pollution because they change the chemical composition of the natural atmosphere (Wisegeek, 2003). Pollution is the introduction of contaminants into a natural environment that causes instability, disorder, harm or discomfort to the ecosystem that is physical system or living organisms. The rate of environmental pollution in developing nations most especially from manufacturing companies or industries is at an alarming rate. Many of the industries release solid particles, water droplets and gases into the atmosphere, while some release their wastes on land or into water bodies. Air pollution is caused by smoke, particulate dust, gaseous emissions, noise among others (Buchanan & Horwitz, 2010).Majority of these wastes are not friendly with human and the environment. They cause harm to both, directly and indirectly.

The process of getting useful stone from a quarry is known as quarrying. The methods and equipment used in quarry depend on the purpose for which the stone is extracted. Different quarrying activities have different impacts on air quality (Babatunde et al., 2013). Mineral exploration, mining and processing have resulted in environmental damages including ecological disturbance; destruction of natural flora; pollution of air, land and water; instability of soil and rock masses; land-scape degradation and radiation hazards due to the emissions of greenhouse gases.

Pollution is the introduction of contaminants into a natural environment that causes instability, disorder, harm or discomfort to the ecosystem that is physical system or living organisms. The rate of environmental pollution in developing nations most especially from manufacturing companies or industries is at an alarming rate. Many of the industries release solid particles, water droplets and gases into the atmosphere, while some release their wastes on land or into water bodies. Air pollution is caused by smoke, particulate dust, gaseous emissions, noise among others (Buchanan &Horwitz, 2010).Majority of these wastes are not friendly with human and the environment. They cause harm to both, directly and indirectly.