# Low Stress ZnO Thin Films for NEMS Device Applications

Jitendra Singh

CSIR-Central Electronics Engineering Research Institute, Pilani, Rajasthan 333031, India

### Abstract

Role of oxygen partial pressure (RO2) on structural, optical and mechanical properties of nano-ZnO films has been investigated. Highly c-axis oriented ZnO thin films in nanometer rage were deposited at room temperature by RF reactive magnetron sputtering. It was found that stress is compressive in nature and varies in -0.06×109 dyne/cm2 to -2.27×109 dyne/cm2 range, as a function of oxygen partial pressure. The stress measurement results are consistent and agreement with X-ray diffraction results. The in-plane residual stress strongly depends on processing parameters and nearly stress free nano-films can be obtained using optimal process parameters. The stressed film shows blue shift in band gap with increase of oxygen partial pressure. The observed variations in mechanical, structural and optical properties are correlated with film stress. Low stress, room temperate deposited piezoelectric nano-ZnO films are highly desirable for NEMS devices. Further, stresses relaxed ZnO thin film based devices are highly reliable, reproducible and exhibit enhanced performance. Yield and device life time can be improved by controlling film stress. In-plane stress primarily produced in ZnO thin films due to conditions imposed by underlying substrate and deposition process. Large in-plane stress can develop in ZnO thin films due to intrinsic or thermal stress. As known, MEMS devices are constituted of mechanical structures and composed of metal, dielectric and functional layers. The ZnO processing temperature must be lower to avoid multilayer inter-diffusion. Sharp, clean and crack free interfaces are highly desirable in NEMS. When NEMS devices exposed to higher temperature, there are possibilities to develop cracks due to mismatch in thermal coefficient of expansion (TCE) of constituent layers. Therefore, there is a need of low process temperature and stress relaxed ZnO film deposition which is established in present investigation.

### Image

Fig. 1: X-ray diffraction of ZnO film prepared in the 30-70% oxygen content. Inset shows the 2θ peak shift with oxygen content.

Recent Publications

- G. E. Gardeniers, Z.M. Rittersma and G.J. Burger, J. Appl. Phys. 83, 7844 (1998).

- R. Menon, V. Gupta, H.H. Tan, K. Sreenivas and C. Jagadish J. Appl. Phys. 109, 064905 (2011).

- R. Menon, K. Sreenivas and V. Gupta, J. Appl. Phys. 103, 094903 (2008).

- R. Singh, M. Kumar and S. Chandra, J. Mater. Sci. 42, 4675 (2007).

- Jitendra Singh, Sapana Ranwa, Jamil Akhtar and Mahesh Kumar, AIP Advances 5, 067140 (2015).

Biography

Jitendra Singh received M.Tech. degree in 2006 from IIT Delhi. Further, he joined Central Research Laboratory, Bharat Electronics Ltd. (BEL) Bangalore and worked till 2010. Currently, he is a scientist in CSIR-CEERI Pilani. He has completed Ph.D. degree in 2016 from AcSIR, New delhi. He has also worked in IISc Bangalore under joint research programme. He has published nearly 50 research papers in journals and conference proceedings.

Email: jitendra@ceeri.res.in

Notes/Comments: